HK Segment Wagon

Longitudinal

Forward & Reverse inputs drive carriage (FWD & REV). 2nd contact on either movement is to engage fast motion. Fast motion is only enabled once in set zone (i.e. encoder position). All movement will be interrupted by e-stop (enabling STO on VSD) Both ends of rail will have a slow zone & a stop (all zones set via encoder position). All carriage motors are braked & will need parameters set to activate coil of braking contactor once movement signal is received. The reset push button will act as a Horn as well to indicate movement & will activate a flashing light upon movement commencing.

Rotation

Rotate left & right enabled by L & R Joystick inputs. 2nd contact on either movement is to engage fast motion. Fast motion is only enabled once in set zone & position is monitored by encoder. All movement will be interrupted by e-stop (enabling STO on VSD).

Emergency Circuit

System will include 2 x emergency stop PB’s (1 on switchboard, 1 on remote) & will also include a maintenance switch on switchboard (isolation switch in series of emergency circuit that can be locked. Once any of these devices are activated, both VSD’s STO function will be enabled & prevent movement. This will require a manual reset (either via the push button on the remote or via a button on the HMI.

HMI

HMI will include a releases page to indicate status of all devices with feedback. To have separate columns for (Rotator, Carriage motors) or separate pages, yet to be decided.

There is a page with carriage & rotator motors that change colour dependant on state (Grey=Not Active, Green=Running & Red=Fault)

Home page will have buttons to navigate to above pages. There will be a Motor Speed parameter setpoint of both motors. There will be an incremental increase/decrease button to add the width of a segment once a ring has been completed. There will be a zero/home button to zero position of carriage.