FUNCTIONING SPECIFICATION

CATHODE HANDLING SYSTEM

FREYR

| CODE: | 217108-EFD-001 |
| --- | --- |
| REVISION: | 01 |
| DATE: | 30.04.2022 |
| ISSUED BY: | Marco Gagliano |
| VERIFIED BY: | Marco Ferrari |
| APPROVED BY: | Marco Bartolini |
| TOTAL PAGES: | 111 |
| ANNEXES: | 0 |

TABLE OF CONTENTS

1	Introduction	6

1.1	Brief functioning description	6

1.2	Control system	7

2	Description of the subsystem equipment.	8

2.1	LFP-NCO system big bag unloading station NZ001.	8

2.1.1	Control device instrumentation NZ001.	8

2.2	Hopper.	9

2.2.1	LFP-NCO system Control device instrumentation Hopper TE005, TE009, TE011 and TE019.	9

2.2.2	CB system control device instrumentation Hopper TE009.	12

2.3	Pneumatic transport.	12

2.3.1	LFP-NCO system control device instrumentation pneumatic transport VZ007, V012 and VZ017.	13

2.3.2	CB system control device instrumentation pneumatic transport VZ007.	15

2.4	LFP-NCO system diverter NZ460.	15

2.4.1	Control device instrumentation diverter.	16

2.5	LFP-NCO system milling.	16

2.5.1	Control device instrumentation milling.	17

2.6	Dryer and Cooler.	18

2.6.1	LFP-NCO system control device instrumentation Dryer HX014 and Cooler VK016.	18

2.6.2	CB system control device instrumentation Dryer/Cooler VK006	21

2.7	CB system bag unloading station Glove Box.	22

2.7.1	Control device instrumentation bag unloading station Glove Box.	23

2.8	Magnetic filters.	24

2.8.1	LFP-NCO system control device instrumentation CA021.	24

2.8.2	CB system control device instrumentation CA011.	25

3	Functional description	26

3.1	Mode of Operation and Machine Behavior	26

3.2	State 4. Idle - Machine not running.	28

3.2.1	LFP-NCO system big bag unloading station NZ001.	28

3.2.2	LFP-NCO system Hopper TE005.	28

3.2.3	LFP-NCO system pneumatic transport VZ007	29

3.2.4	LFP-NCO system diverter NZ460.	29

3.2.5	LFP-NCO system Hopper TE009.	29

3.2.6	LFP-NCO system Milling.	30

3.2.7	LFP-NCO system Hopper TE011.	30

3.2.8	LFP-NCO system pneumatic transport VZ012	31

3.2.9	LFP-NCO system Dryer HX014.	31

3.2.10	LFP-NCO system Cooler VK016.	32

3.2.11	LFP-NCO system pneumatic transport VZ017	33

3.2.12	LFP-NCO system Hopper TE019.	33

3.2.13	LFP-NCO system magnetic filters CA021.	34

3.2.14	CB system bag unloading station Glove Box.	34

3.2.15	CB system Dryer/Cooler VK006.	35

3.2.16	CB system pneumatic transport VZ007	36

3.2.17	CB system Hopper TE009.	36

3.2.18	CB system magnetic filters CA011.	36

3.3	State 3. Starting - Machine warm up.	37

3.3.1	LFP-NCO system. Dryer HX014 Starting.	37

3.3.2	LFP-NCO system. Cooler VK016 Starting.	38

3.3.3	CB system. Dryer/Cooler VK006 Starting.	39

3.4	State 6. Execute - Machine Running.	40

3.4.1	LFP-NCO system. Unloading of big bags in automatic sequence from NZ001 to TE005.	40

3.4.2	LFP-NCO system. Unloading from TE005 to VZ007	42

3.4.3	LFP-NCO system. Conveying from VZ007 to TE009 (LFP mode selected)	43

3.4.4	LFP-NCO system. Conveying from VZ007 to TE011 (NMC Mode selected)	45

3.4.5	LFP-NCO system. Unloading of LFP from TE009 to Milling System	46

3.4.6	LFP-NCO system. Sampling of the product after milling phase	47

3.4.7	LFP-NCO system. Unloading from mill to TE011	48

3.4.8	LFP-NCO system. Unloading from TE011 to VZ012	49

3.4.9	LFP-NCO system. Convey from VZ012 to Dryer HX014	50

3.4.10	LFP-NCO system. Drying sequence Dryer HX014	52

3.4.11	LFP-NCO system. Unloading from Dryer HX014 to Cooler VK016	53

3.4.12	LFP-NCO system. Cooler sequence Cooler VK016	56

3.4.13	LFP-NCO system. Unloading from Cooler VK016 to VZ017	58

3.4.14	LFP-NCO system. Conveying from VZ017 to TE019	60

3.4.15	LFP-NCO system. Cleaning of VZ017	61

3.4.16	LFP-NCO system. Unloading from T019 to Slurry Preparation.	62

3.4.17	CB system, bag unloading station Glove Box.	63

3.4.18	CB system. Unloading from station Glove Box to Dryer/Cooler VK006	66

3.4.19	CB system. Dryer/cooler VK006 sequence.	68

3.4.20	CB system. Unloading from Dryer/cooler VK006 to VZ007	70

3.4.21	CB system. Convey from VZ007 to TE009	72

3.4.22	CB system. Unloading from T019 to Slurry Preparation.	73

3.5	State 16. Completing - Almost complete	74

3.5.1	LFP-NCO system. Dryer HX014 Completing.	75

3.5.2	LFP-NCO system. Cooler VK016 Completing.	78

3.5.3	CB system. Dryer/Cooler VK006 Completing.	80

3.6	State 17. Completed - Sequence completed.	83

3.7	State 19. E-Stop - Emergency Stop pressed.	83

3.7.1	LFP-NCO system big bag unloading station NZ001.	84

3.7.2	LFP-NCO system Hopper TE005.	84

3.7.3	LFP-NCO system pneumatic transport VZ007	85

3.7.4	LFP-NCO system diverter NZ460.	85

3.7.5	LFP-NCO system Hopper TE009.	85

3.7.6	LFP-NCO system Milling.	86

3.7.7	LFP-NCO system Hopper TE011.	86

3.7.8	LFP-NCO system pneumatic transport VZ012	87

3.7.9	LFP-NCO system Dryer HX014.	87

3.7.10	LFP-NCO system Cooler VK016.	88

3.7.11	LFP-NCO system pneumatic transport VZ017	89

3.7.12	LFP-NCO system Hopper TE019.	89

3.7.13	LFP-NCO system magnetic filters CA021.	90

3.7.14	CB system bag unloading station Glove Box.	90

3.7.15	CB system Dryer/Cooler VK006.	91

3.7.16	CB system pneumatic transport VZ007	92

3.7.17	CB system Hopper TE009.	92

3.7.18	CB system magnetic filters CA011.	92

3.8	Batching System	93

3.8.1	LFP System Weight Batching	93

3.8.2	NMC Batching System	94

4	Alarms	96

4.1	Immediate shutdown	96

4.2	Controlled shutdown	98

4.3	Warnings	98

4.4	Interlocks	99

5	Process Settings and Alarm Settings	106

5.1	Transporter Setting	106

5.1.1	Process settings	106

5.1.2	Alarm Settings	106

5.2	Diverter Settings	107

5.3	Fluidization PAD settings	107

5.4	Dryer, Cooler and Dryer/Cooler Settings	107

5.5	Dryer, Cooler and Dryer/Cooler Alarms Settings.	109

5.6	Dryer, Cooler and Dryer/Cooler Alarms	110

# Introduction

This document describes the functioning cycles and the abnormal sequences of the cathode handling system supplied by NTE for the FREYR pilot plant.

The flow diagrams 217108-PID-002 and 217108-PID-004 should be referenced while reading this specification.

## Brief functioning description

The handling system is divided into two sub-system and is composed by the following equipment:

For the LFP-NCO system:

- LFP- NCO big bag unloading station (NZ001). This equipment receives the material that is uploaded by the operator.

- Hopper TE005. Once the material has been scanned and located in the input equipment, this hopper receives it and weights it for process monitoring.

- Transporter VZ007. The VZ007 transporter is capable of transferring material from the TE005 hopper to the TE009 hopper if the configuration dictates that milling is necessary. Otherwise, it will bypass the milling process and transport the material directly to the TE011 hopper.

- Hopper TE009. The ET009 hopper receives the product and weighs it for traceability purposes. This hopper serves as the point at which the material is held prior to entering the mill.

- Milling. This equipment, as its title implies, is designed to mill material. Depending on the outcome of the operational assessment, the material can proceed to the next step of processing or be discarded.

- Hopper TE011. The hopper receives the product from the mill or from the transporter VZ007 depending on the configuration of the product and is weighed and analyzed for production monitoring.

- Transporter VZ012. The transporter is designed to facilitate the transfer of the product from TE011 hopper to the HX014 drying system.

- Heater/Dryer HX014. Once the material has been processed to this stage, this equipment has the capability to raise the temperature of the product and dry it, based on the settings of the machine.

- Cooler VK016. When the material has dried, it reaches a high temperature, so cooling is necessary to proceed. This equipment is designed to handle this task.

- Transporter VZ017. The VZ017 transporter is designed to receive material from the cooler and transport it to the ET019 hopper.

- Hopper TE019. The ET019 hopper is the final repository for processed material, which is arranged for operational purposes. It is necessary to weigh and analyze the material or product for production monitoring, as with all hoppers.

For the CB system:

- CB bag unloading station. This equipment is designed to safely and effectively handle the disposal of materials introduced into the system, as well as any waste generated as a result.

- Dryer/cooler VK006. When the material enters this equipment, it has the function of drying and cooling the material depending on the production configuration.

- Smartjet VZ007. Once the material has been processed in VK006, it is transferred to the VZ007 Smartjet for transport to the TE009 hopper of this section.

- Hopper TE009 with circle feeder. The material is received in this hopper, which is weighed and monitored for operational purposes. At this point the material is stored and is available for operation when required.

The purpose is to handle the LFP and the CB and, after a milling process and a drying/cooling process to transport the product to the slurry preparation system.

## Control system

The PLC used for the LFP and the CB system is a Siemens 1517F, along with a HMI UNIFIED COMFORT panel that will give to the operator all the events and the status of each sequence / devices. It is also foreseen an exchange data process trough OPC UA protocol to the MES system.

# Description of the subsystem equipment.

The subsystem comprises a variety of equipment, each with its own purpose and operation. This includes pumps, valves, motors, and other related components.

## LFP-NCO system big bag unloading station NZ001.

This NZ001 is a single-entry system to the TE005 hopper. The description in the following chapters is referred to NZ001, please refer to P&ID for actual tag numbers of components.

With the help of an overhead crane, big bags are loaded onto the big bag unloading station (NZ001), and, once the bag is loaded System to carry out the following operations automatically.

The station is fitted with an automatic latch door blocking/enabling the product into the hopper.

To enable the opening latch door, the operator must successfully scan the barcode of the big bag.

Once the bag is opened, and the door opens, the product will flow into the underlying hopper for transport preparation. The operator must close the door right after the product starts flowing in order to minimize the quantity of dust emissions.

The hopper fills itself with product, once the top-level sensor of the hopper has been reached or the weight is reached, the operator can proceed to lift and remove the big bag, preparing to load the next one onto the NZ001.

### Control device instrumentation NZ001.

The following table shows the NZ001 devices.

| Device | Tag | Description |
| --- | --- | --- |
| GK002 (Motor) | GK002-M01-VFD | Aspiration fan |
| HY025 (Valve) | AY025 | Solenoid-vibrator solenoid valves |
| HY028 (Valve) | AY028 | Solenoid-The piston solenoid valve gate |
| HY031 (Valve) | AY031 | Solenoid-Spare |
| HY035 (Valve) | AY034 | Solenoid-Spare |
| HY097 (Valve) | AY097 | Solenoid-inject pressure to NZ001 |
| PDIT086 (H) | PDIT086 | Sensor-Inlet Aspiration fan |
| PIT128 (H) | PIT128 | Sensor-Pressure transmitter (inside NZ001) |
| LP036 (Valve) | ZSL037 | Sensor- Closed valve LP036 position confirmation |
| LP036 (Valve) | ZSH038 | Sensor- Open valve LP036 position confirmation |
| LP036 (Valve) | AY035 | Solenoid-valve-Inlet valve of the hopper TE005 |

## Hopper.

The hopper is used as a buffer between the equipment and the pneumatic transport system downstream of it. When empty it will allow the product to be unloaded from the other equipment, and then transferred downstream.

The hopper is fitted with a fluidization system allowing for proper aeration of the product and easy movement of the latter. Fluidization is typically enabled when the product is unloading and also when the hopper has some product in it, and at rest, albeit with a different setting. They are fitted with a filter that gathers the dust generated during transport. High- and low-level sensors are used during loading and unloading sequences of the hopper. A butterfly valve or a screw conveyor enables unloading of the hopper into the component downstream of it.

### LFP-NCO system Control device instrumentation Hopper TE005, TE009, TE011 and TE019.

The following table shows the TE005 devices.

| Device | Tag | Description |
| --- | --- | --- |
| CA004 (Filter) | KC047 | Run Filter |
| HY054 (Valve) | AY054 | Solenoid-Fluidization |
| HY055 (Valve) | AY055 | Solenoid-Fluidization |
| AMOT-M01(Motor) | NC006-M01 | Motor-Screw Conveyor |
| AMOT-M02(Motor) | NC006-M02-VFD | Motor-Screw Conveyor |
| HY443 (Valve) | AY443 | Solenoid-Vertical Spout with PN closing |
| HY056 (Valve) | AY056 | Solenoid-inject pressure to NC006 |
| CA004-PDIT046 (H) | PDIT046 | Sensor-Filter CA004 pressure difference transmitter |
| PDIT135 (H) | PDIT135 | Sensor-Filter CA004 outlet pressure difference transmitter |
| PS051 (H) | PS051 | Sensor-Hopper outlet pressure switch (Safety) |
| LS039 (H) | LS039 | Sensor-High level switch of the hopper TE005 |
| LS092 (L) | LS092 | Sensor-Low level switch of the hopper TE005 |
| PIT040 (H) | PIT040 | Sensor-Hi Pressure transmitter (inside TE005) |
| WT045 | WT045 | Sensor-weight transmitter (scale) |
| AY050 | AY050 | Indicator-Top Rupture Disk (Safety) |
| AY052 | AY052 | Indicator-Top Rupture Disk (Safety) |
| AY065 | AY065 | Indicator-Top Rupture Disk (Safety) |

The following table shows the TE009 devices.

| Device | Tag | Description |
| --- | --- | --- |
| CA008 (Filter) | KC118 | Run Filter |
| HY133 (Valve) | AY133 | Solenoid-Fluidization |
| HY135 (Valve) | AY135 | Solenoid-Fluidization |
| LP145 (Valve) | AY144 | Solenoid-valve-Outlet valve of the TE009 |
| LP145 (Valve) | ZSL146 | Sensor-Closed valve LP145 position confirmation |
| LP145 (Valve) | ZSH147 | Sensor-Open valve LP145 position confirmation |
| CA008-PDIT117 (H) | PDIT117 | Sensor-Filter CA008 pressure difference transmitter |
| PDIT138 (H) | PDIT138 | Sensor-Filter CA008 outlet pressure difference transmitter |
| PS115 (H) | PS115 | Sensor-Hopper outlet pressure switch (Safety) |
| LS123 (H) | LS123 | Sensor-High level switch of the hopper TE009 |
| LS137 (L) | LS137 | Sensor-Low level switch of the hopper TE009 |
| PIT124 (H) | PIT124 | Sensor-Hi Pressure transmitter (inside TE009) |
| WT129 | WT129 | Sensor-weight transmitter (scale) |
| AY116 | AY116 | Indicator-Top Rupture Disk (Safety) |
| AY114 | AY114 | Indicator-Top Rupture Disk (Safety) |

The following table shows the TE011 devices.

| Device | Tag | Description |
| --- | --- | --- |
| CA010 (Filter) | KC156 | Run Filter |
| HY171 (Valve) | AY171 | Solenoid-Fluidization |
| HY172 (Valve) | AY172 | Solenoid-Fluidization |
| LP175 (Valve) | AY174 | Solenoid-valve-Outlet valve of the TE011 |
| LP175 (Valve) | ZSL177 | Sensor-Closed valve LP175 position confirmation |
| LP175 (Valve) | ZSH176 | Sensor-Open valve LP175 position confirmation |
| CA010-PDIT157 (H) | PDIT157 | Sensor-Filter CA008 pressure difference transmitter |
| PDIT140 (H) | PDIT140 | Sensor-Filter CA008 outlet pressure difference transmitter |
| PS159 (H) | PS159 | Sensor-Hopper outlet pressure switch (Safety) |
| LS161 (H) | LS161 | Sensor-High level switch of the hopper TE011 |
| LS218 (L) | LS218 | Sensor-Low level switch of the hopper TE011 |
| PIT162 (H) | PIT162 | Sensor-Hi Pressure transmitter (inside TE011) |
| WT166 | WT166 | Sensor-weight transmitter (scale) |
| AY158 | AY158 | Indicator-Top Rupture Disk (Safety) |
| AY160 | AY160 | Indicator-Top Rupture Disk (Safety) |
| AY180 | AY180 | Indicator-Top Rupture Disk (Safety) |

The following table shows the TE019 devices.

| Device | Tag | Description |
| --- | --- | --- |
| CA018 (Filter) | KC414 | Run Filter |
| HY435 (Valve) | AY435 | Solenoid-Fluidization |
| HY436 (Valve) | AY436 | Solenoid-Fluidization |
| AMOT-M01(Motor) | NC020-M01 | Motor-Screw Conveyor |
| AMOT-M02(Motor) | NC020-M02-VFD | Motor-Screw Conveyor |
| HY438 (Valve) | AY438 | Solenoid-inject pressure to NC020 |
| CA018-PDIT413 (H) | PDIT413 | Sensor-Filter CA018 pressure difference transmitter |
| PDIT209 (H) | PDIT209 | Sensor-Filter CA018 outlet pressure difference transmitter |
| PS409 (H) | PS409 | Sensor-Hopper outlet pressure switch (Safety) |
| LS415 (H) | LS415 | Sensor-High level switch of the hopper TE018 |
| LS421 (L) | LS421 | Sensor-Low level switch of the hopper TE018 |
| PIT416 (H) | PIT416 | Sensor-Hi Pressure transmitter (inside TE018) |
| WT421 | WT421 | Sensor-weight transmitter (scale) |
| AY408 | AY408 | Indicator-Top Rupture Disk (Safety) |
| AY410 | AY410 | Indicator-Top Rupture Disk (Safety) |

### CB system control device instrumentation Hopper TE009.

The following table shows the TE009 devices.

| Device | Tag | Description |
| --- | --- | --- |
| CA008 (Filter) | KC001 | Run Filter |
| AMOT-M01(Motor) | NC010-M01-VFD | Motor-rotational unloader |
| LP007 (Valve) | AY020 | Solenoid-Outlet valve of the TE009 |
| LP007 (Valve) | ZSL010 | Sensor-Closed valve LP007 position confirmation |
| LP007 (Valve) | ZSH010 | Sensor-Open valve LP007 position confirmation |
| CA018-2-PDIT002 (H) | PDIT002 | Sensor-Filter CA008 pressure difference transmitter |
| PDIT004 (H) | PDIT004 | Sensor-Filter CA008 outlet pressure difference transmitter |
| LS003 (H) | LS003 | Sensor-High level of the hopper TE009 |
| LS001 (L) | LS001 | Sensor-Low level of the hopper TE009 |
| PIT002 (H) | PIT002 | Sensor-Hi Pressure transmitter (inside TE009) |
| WT001 | WT001 | Sensor-weight transmitter (scale) |

## Pneumatic transport.

The description in the following section refers to the transporter, see the P&ID for actual component tag numbers.

In order for the auto-cycle to run for a given bin, the bin must be enabled, and the auto-cycle must be running for that bin.

When one of its destinations or bins requests material, the vessel begins to load. When the high level or set operating weight is reached, the container valves close. If the line is free, the boat will mark the line and the diverter valves will move if the line is diverted. When the bypass valves are in position, the vessel is pressurized. The material is transported to the equipment or hopper. When the low-pressure level is reached, the conveyor stops conveying by relieving removing supply to the transport valve, and opening the vent valve. A new charge cycle will start until the target high level is reached, or the target weight has been reached. In semi-automatic mode, the sequences of the transportation cycles can be tested one by one until the target weight is reached, or the high level is reached in the downstream equipment. This mode should be used for testing, commissioning, and maintenance purposes only.

In manual mode, all devices can be tested independently.

### LFP-NCO system control device instrumentation pneumatic transport VZ007, V012 and VZ017.

The following table shows the VZ007 devices.

| Device | Tag | Description |
| --- | --- | --- |
| LP059 (Valve) | AY062 | Solenoid-Inlet valve of the transporter |
| LP059 (Valve) | ZSL061 | Sensor-Closed valve LP059 position confirmation |
| LP059 (Valve) | ZSH060 | Sensor-Open valve LP059 position confirmation |
| LP066 (Valve) | AY069A | Solenoid-VZ007 Pressure release valve open |
| LP066 (Valve) | AY069B | Solenoid-VZ007 Pressure release valve closed |
| LP066 (Valve) | ZSL068 | Sensor-Closed valve LP066 position confirmation |
| LP066 (Valve) | ZSH067 | Sensor-Open valve LP066 position confirmation |
| PRV077 | ZIC077 | Proportional Valve- Control pressure line |
| PRV078 | ZIC078 | Proportional Valve- Control pressure vessel |
| PRV077 | ZIT077 | Feedback- Control pressure line |
| PRV078 | ZIT078 | Feedback- Control pressure vessel |
| LP088 (Valve) | AY091 | Solenoid-Outlet valve of the transporter |
| LP088 (Valve) | ZSL061 | Sensor- Closed valve LP088 position confirmation |
| LP088 (Valve) | ZSH060 | Sensor-Open valve LP088 position confirmation |
| PS064 (H) | PS064 | Sensor- Vessel outlet pressure switch (Safety) |
| LS070 (H) | LS070 | Sensor-High level switch of the vessel |
| PIT075 (H/L) | PIT075 | Sensor-Pressure transmitter (inside VZ007) |
| WT087 | WT087 | Sensor-weight transmitter (scale) |
| AY063 | AY063 | Indicator-Top Rupture Disk (Safety) |

The following table shows the VZ012 devices.

| Device | Tag | Description |
| --- | --- | --- |
| LP195 (Valve) | AY182 | Solenoid-Inlet valve of the transporter |
| LP195 (Valve) | ZSL205 | Sensor-Closed valve LP195 position confirmation |
| LP195 (Valve) | ZSH204 | Sensor-Open valve LP195 position confirmation |
| LP181 (Valve) | AY181A | Solenoid-VZ012 Pressure release valve open |
| LP181 (Valve) | AY181B | Solenoid-VZ012 Pressure release valve closed |
| LP181 (Valve) | ZSL204 | Sensor-Closed valve LP181 position confirmation |
| LP181 (Valve) | ZSH203 | Sensor-Open valve LP181 position confirmation |
| PRV196 | ZIC196 | Proportional Valve- Control pressure line |
| PRV197 | ZIC197 | Proportional Valve- Control pressure vessel |
| PRV196 | ZIT196 | Feedback- Control pressure line |
| PRV197 | ZIT197 | Feedback- Control pressure vessel |
| LP138 (Valve) | AY140 | Solenoid-Outlet valve of the transporter |
| LP138 (Valve) | ZSL191 | Sensor- Closed valve LP138 position confirmation |
| LP138 (Valve) | ZSH190 | Sensor-Open valve LP138 position confirmation |
| PS179 (H) | PS179 | Sensor- Vessel outlet pressure switch (Safety) |
| LS183 (H) | LS183 | Sensor-High level switch of the vessel |
| PIT194 (H/L) | PIT194 | Sensor-Pressure transmitter (inside VZ012) |
| WT210 | WT210 | Sensor-weight transmitter (scale) |
| AY178 | AY178 | Indicator-Top Rupture Disk (Safety) |

The following table shows the VZ017 devices.

| Device | Tag | Description |
| --- | --- | --- |
| LP363 (Valve) | AY366 | Solenoid-Inlet valve of the transporter |
| LP363 (Valve) | ZSL365 | Sensor-Closed valve LP363 position confirmation |
| LP363 (Valve) | ZSH364 | Sensor-Open valve LP363 position confirmation |
| LP370 (Valve) | AY371A | Solenoid-VZ017 Pressure release valve open |
| LP370 (Valve) | AY371B | Solenoid-VZ017 Pressure release valve closed |
| LP370 (Valve) | ZSL459 | Sensor-Closed valve LP370 position confirmation |
| LP370 (Valve) | ZSH458 | Sensor-Open valve LP370 position confirmation |
| LP485 (Valve) | AY490 | Solenoid-Equalize pressure with VX016 |
| LP485 (Valve) | ZSL487 | Sensor-Closed valve LP485 position confirmation |
| LP485 (Valve) | ZSH486 | Sensor-Open valve LP485 position confirmation |
| HY384 (Valve) | AY384 | Solenoid-Control pressure line and vessel |
| HY378 (Valve) | AY378 | Solenoid- Cleaning from the top |
| PS368 (H) | PS368 | Sensor- Vessel outlet pressure switch (Safety) |
| LS372 (H) | LS372 | Sensor-High level switch of the vessel |
| PIT383 (H/L) | PIT383 | Sensor-Pressure transmitter (inside VZ012) |
| AY367 | AY367 | Indicator-Top Rupture Disk (Safety) |
| AY369 | AY369 | Indicator-Top Rupture Disk (Safety) |

### CB system control device instrumentation pneumatic transport VZ007.

The following table shows the VZ007 devices.

| Device | Tag | Description |
| --- | --- | --- |
| LP001 (Valve) | AY006 (Open Solenoid) | Solenoid-Inlet valve of the transporter |
| LP001 (Valve) | ZSL005 | Sensor- Closed valve LP001 position confirmation |
| LP001 (Valve) | ZSH005 | Sensor- Open valve LP001 position confirmation |
| HY005 (Valve) | AY005 | Solenoid-inject pressure N. to VZ007 |
| LS002 (H) | LS002 | Sensor-High level switch of the vessel |
| PS001 (H/L) | PS001 | Sensor- Inlet VZ007 pressure N. switch |
| PS003 (H) | PS003 | Sensor- VZ007 Inlet pressure switch (Safety) |
| AY036 | AY036 | Sensor- Top Rupture Disk (Safety) |
| AY034 | AY034 | Sensor- Top Rupture Disk (Safety) |

## LFP-NCO system diverter NZ460.

Destination of product in the transporters VZ007 is changed by switching the position of the diverter valve. The plug type diverter consists of an aluminum housing and end covers in which a plug with two pipes is installed. A cylinder is used to control the opening/closing of the plugs and an inflatable gasket is used when conveying.

Normally the gasket is deflated. This is checked before changing the position of the diverter, by verifying the status of the pressure switch.

Before any conveying is done, the gasket is inflated. To inflate the gasket the solenoid energized. The correct inflation is verified against the pressure switch and the position proximity feedback is verified before conveying is started.

### Control device instrumentation diverter.

The following table shows the NZ460 devices.

| Device | Tag | Description |
| --- | --- | --- |
| HY463 (Valve) | AY463 | Solenoid-Deviator Valve LFP |
| HY464 (Valve) | AY464 | Solenoid-Deviator Valve NMC |
| HY466 (Valve) | AY466 | Solenoid-Increase pressure in NZ460 gasket inflated |
| NZ460 | ZSL461 | Sensor-Deviator Valve NZ460 in straight position |
| NZ460 | ZSH462 | Sensor-Deviator Valve NZ460 in diverted position |
| PSH465 | PSH465 | Sensor-inject pressure switch gasket inflated |

## LFP-NCO system milling.

The milling system, as previously mentioned, fulfills the function of milling material for production. Although not under full control of the system, it communicates with the NTE PLC via hardwired contacts. Here below is described a table which resumes the signal exchanging.

For the charge powder process to the milling station, the following signals exchanging have been foreseen.

| FROM | TO | DESCRIPTION |
| --- | --- | --- |
| NCE | NTE | CP In normal operation |
| NCE | NTE | NTE can charge powder. |
| NCE | NTE | NTE can not charge powder. |
| NCE | NTE | CP Discharge Valve Open |
| NCE | NTE | CP Discharge Valve Close |
| NCE | NTE | CP requires powder. |
| NCE | NTE | CP is ready to discharge powder. |
| NCE | NTE | CP is discharging powder. |

For the powder discharge process, from the milling system to the TE011, the following signals have been foreseen.

| FROM | TO | DESCRIPTION |
| --- | --- | --- |
| NTE | NCE | NTE SYSTEM In normal operation |
| NTE | NCE | NTE1 Valve Open |
| NTE | NCE | NTE1 Valve Close |
| NTE | NCE | NTE2 Valve Open |
| NTE | NCE | NTE2 Valve Close |
| NTE | NCE | NTE1 is ready to charge powder. |
| NTE | NCE | NTE1 is charging powder. |
| NTE | NCE | NTE2 requires powder. |
| NTE | NCE | NTE2 have completed the sampling of the product. |

Note: tag number for these signals are yet to be defined.

### Control device instrumentation milling.

In order to ensure proper grinding, a sample cylinder and a series of valves are to be utilized. By taking a sample and running it through the valves, the operation can either accept or reject the production of the material. The equipment used is outlined in the below table.

| Device | Tag | Description |
| --- | --- | --- |
| HY475A/B | AY475A | Solenoid-Piston extension (sampling cylinder) |
| HY475A/B | AY475B | Solenoid-Piston retraction (sampling cylinder) |
| HY475A/B | ZSL479 | Sensor-Piston retraction position confirmation |
| HY475A/B | ZSH480 | Sensor-Piston extension position confirmation |
| HY476A/B | AY476A | Solenoid-Rotation of the sampling cylinder |
| HY476A/B | AY476B | Solenoid-Rotation of sampler in rest position |
| HY473 | AY473 | Solenoid-Gasket inflation |
| HY474 | AY474 | Solenoid-Gasket inflation |
| PS472 (L) | PS472 | Sensor-Gasket inflation pressure switch |
| LP478 (Valve) | AY477 | Solenoid-Outlet valve of the milling |
| LP478 (Valve) | ZSL481 | Sensor-Closed valve LP478 position confirmation |
| LP478 (Valve) | ZSH482 | Sensor-Open valve LP478 position confirmation |
| NZ216 | AY148A | Solenoid-NZ216 Straight position |
| NZ216 | AY148B | Solenoid-NZ216 Reject position |

## Dryer and Cooler.

The description in the following section refers to the dryer and cooler, see the P&ID for actual component tag numbers.

The primary purpose of the drying system is to raise the temperature of the product during the drying process. The system features a jacketed container with a serpentine pipe, through which high-temperature oil flows, to evenly distribute the heat to the material via an agitator until the desired drying point is reached. To remove moisture, the system is equipped with a vacuum pump, which is protected by filters to prevent the material from being sucked through. The cooling system operates on the same principle, with the only difference being that the fluid passing through the serpentine pipe is cooling water.

### LFP-NCO system control device instrumentation Dryer HX014 and Cooler VK016.

The following table shows the HX014 devices.

| Device | Tag | Description |
| --- | --- | --- |
| CA013 (Filter) | KC235 | Run Filter |
| LP236 (Valve) | AY337A | Solenoid-Outlet filter CA013 valve open |
| LP236 (Valve) | AY337B | Solenoid-Outlet filter CA013 valve close |
| LP236 (Valve) | ZSL238 | Sensor-Closed valve LP236 position confirmation |
| LP236 (Valve) | ZSH237 | Sensor-Open valve LP236 position confirmation |
| CA013-PDIT234 (H) | PDIT234 | Sensor-Filter CA013 pressure difference transmitter |
| CA013-PDIT141 (H) | PDIT141 | Sensor-Filter CA013 outlet pressure difference transmitter |
| CA013-LS142 (H) | LS142 | Sensor-High level of the Filter CA013 |
| CA013-TIT233 (H) | TIT233 | Sensor-Filter CA013 Temperature transmitter |
| LP222 (Valve) | AY223 | Solenoid-inject pressure to the bag filter unit from VX006 |
| LP225 (Valve) | AY226 | Solenoid-inject pressure to the bag filter unit from VX006 |
| HY447 (Valve) | AY447 | Solenoid-Outlet VX006 cleaning Blades Riboccone |
| PSV279-HY457 (Valve) | AY457 | Solenoid-Outlet Bag filter auto release PDIT280>atm |
| LP356 (Valve) | AY362 | Solenoid-Outlet Bag filter vacuum breaker solenoid valve |
| LP356 (Valve) | ZSL491 | Sensor-Closed valve LP356 position confirmation |
| LP356 (Valve) | ZSH492 | Sensor-Open valve LP356 position confirmation |
| LP357 (Valve) | AY360 | Solenoid-Vacuum breaker nitrogen line |
| LP357 (Valve) | ZSL358 | Sensor-Closed valve LP357 position confirmation |
| LP357 (Valve) | ZSH359 | Sensor-Open valve LP357 position confirmation |
| LP352 (Valve) | AY351 | Solenoid-Vacuum breaker Silencer line |
| LP352 (Valve) | ZSL353 | Sensor- Closed valve LP352 position confirmation |
| LP352 (Valve) | ZSH354 | Sensor-Open valve LP352 position confirmation |
| PRV346 | ZIC346 | Proportional Valve- Outlet Bag filter Vacuum pressure Control |
| PRV346 | ZIT346 | Feedback- Outlet Bag filter Vacuum pressure Control |
| LP347 (Valve) | AY350 | Solenoid-Vacuum pressure Control nitrogen line (?) |
| LP347 (Valve) | ZSL358 | Sensor- Closed valve LP347 position confirmation |
| LP347 (Valve) | ZSH359 | Sensor-Open valve LP347 position confirmation |
| LP342 (Valve) | AY341 | Solenoid-Vacuum pressure Control Silencer line |
| LP342 (Valve) | ZSL353 | Sensor- Closed valve LP342 position confirmation |
| LP342 (Valve) | ZSH354 | Sensor-Open valve LP342 position confirmation |
| AMOT-M03(Motor) | PG398-M03-VFD | Motor-Vacuum pump |
| LP239 (Valve) | AY336A | Solenoid-Material Inlet valve open |
| LP239 (Valve) | AY336B | Solenoid-Material Inlet valve close |
| LP239 (Valve) | ZSL241 | Sensor-Closed valve LP239 position confirmation |
| LP239 (Valve) | ZSH240 | Sensor-Open valve LP239 position confirmation |
| AMOT-M01(Motor) | HX014-M01-VFD | Motor-Rotating drive |
| HY448 (Valve) | AY448 | Solenoid-Cleaning Blades Riboccone |
| HY449 (Valve) | AY449 | Solenoid-Cleaning with Gas from Below |
| HY450 (Valve) | AY450 | Solenoid-Cleaning Exhaust valve gasket |
| HY451 (Valve) | AY451 | Solenoid-Cleaning Exhaust valve gasket |
| HY335 (Valve) | AY335 | Solenoid-Air knocker ctrl sys |
| HY333A/B (Valve) | AY333A | Solenoid-Discharge valve Open position |
| HY333A/B (Valve) | AY333B | Solenoid-Discharge valve Close position |
| HY333A/B (Valve) | ZSL262 | Sensor- Closed Discharge valve position confirmation |
| HY333A/B (Valve) | ZSH263 | Sensor- Open Discharge valve position confirmation |
| HY259 (Valve) | AY259 | Solenoid-inject pressure to the Screw |
| AMOT-M01(Motor) | NC015-M01-VFD | Motor-Screw Conveyor |
| HX014-PDIT280 (H) | PDIT280 | Sensor- Bag filter pressure difference transmitter |
| TIT246 (H) | TIT246 | Sensor- Process temperature sensor transmitter |
| AIT281 -AY281 (H) | AY281 | Indicator-Outlet Bag filter Control unit (Zirconia oxygen analyzer) |

The following table shows the VK016 devices.

| Device | Tag | Description |
| --- | --- | --- |
| LP269 (Valve) | AY270 | Solenoid-inject pressure to the bag filter unit from VX008 |
| LP271 (Valve) | AY272 | Solenoid-inject pressure to the bag filter unit from VX008 |
| HY452 (Valve) | AY452 | Solenoid-Outlet VX008 cleaning Blades Riboccone |
| PSV275-HY242 (Valve) | AY242 | Solenoid-Outlet Bag filter auto release PDIT276>atm |
| LP322 (Valve) | AY328 | Solenoid-Outlet Bag filter vacuum breaker solenoid valve |
| LP322 (Valve) | ZSL493 | Sensor-Closed valve LP322 position confirmation |
| LP322 (Valve) | ZSH494 | Sensor-Open valve LP322 position confirmation |
| LP323 (Valve) | AY326 | Solenoid-Vacuum breaker nitrogen line |
| LP323 (Valve) | ZSL324 | Sensor-Closed valve LP323 position confirmation |
| LP323 (Valve) | ZSH325 | Sensor-Open valve LP323 position confirmation |
| LP318 (Valve) | AY317 | Solenoid-Vacuum breaker Silencer line |
| LP318 (Valve) | ZSL319 | Sensor- Closed valve LP318 position confirmation |
| LP318 (Valve) | ZSH320 | Sensor-Open valve LP318 position confirmation |
| PRV313 | ZIC495 | Proportional Valve- Outlet Bag filter Vacuum pressure Control |
| PRV313 | ZIT495 | Feedback- Outlet Bag filter Vacuum pressure Control |
| LP314 (Valve) | AY316 | Solenoid-Vacuum pressure Control nitrogen line (?) |
| LP314 (Valve) | ZSL315 | Sensor- Closed valve LP314 position confirmation |
| LP314 (Valve) | ZSH314 | Sensor-Open valve LP314 position confirmation |
| LP309 (Valve) | AY308 | Solenoid-Vacuum pressure Control Silencer line |
| LP309 (Valve) | ZSL311 | Sensor- Closed valve LP309 position confirmation |
| LP309 (Valve) | ZSH310 | Sensor-Open valve LP309 position confirmation |
| AMOT-M04(Motor) | PG400-M04-VFD | Motor-Vacuum pump |
| LP401 (Valve) | AY404 | Solenoid-Inlet valve of the cooler Vacuum pump |
| LP401 (Valve) | ZSL402 | Sensor-Closed valve LP401 position confirmation |
| LP401 (Valve) | ZSH403 | Sensor-Open valve LP401 position confirmation |
| LP264 (Valve) | AY339 | Solenoid-Material Inlet valve open |
| LP264 (Valve) | ZSL265 | Sensor-Closed valve LP264 position confirmation |
| LP264 (Valve) | ZSH266 | Sensor-Open valve LP264 position confirmation |
| AMOT-M03(Motor) | VK016-M01-VFD | Motor-Rotating drive |
| HY453 (Valve) | AY453 | Solenoid-Cleaning Blades Riboccone |
| HY454 (Valve) | AY454 | Solenoid-Cleaning with Gas from Below |
| HY455 (Valve) | AY455 | Solenoid-Cleaning Exhaust valve gasket |
| HY456 (Valve) | AY456 | Solenoid-Cleaning Exhaust valve gasket |
| HY304 (Valve) | AY304 | Solenoid-Air knocker ctrl sys |
| HY302A/B (Valve) | AY302A | Solenoid-Discharge valve Open position |
| HY302A/B (Valve) | AY302B | Solenoid-Discharge valve Close position |
| HY302A/B (Valve) | ZSL297 | Sensor- Closed Discharge valve position confirmation |
| HY302A/B (Valve) | ZSH298 | Sensor- Open Discharge valve position confirmation |
| VK016-PDIT276 (H) | PDIT276 | Sensor- Bag filter pressure difference transmitter |
| TIT285 (H) | TIT285 | Sensor- Process temperature sensor transmitter |
| AIT277 -AY277 (H) | AY277 | Indicator-Outlet Bag filter Control unit (Zirconia oxygen analyzer) |
| PG398 (Pump) | LSL001 | H20 Refill Low Level |
| PG398 (Pump) | LSH001 | H20 Refill High Level |

### CB system control device instrumentation Dryer/Cooler VK006

The following table shows the VK016 devices.

| Device | Tag | Description |
| --- | --- | --- |
| LP006 (Valve) | AY003 | Solenoid- Inject pressure to the bag filter unit |
| HY028 (Valve) | AY028 | Solenoid- Cleaning Blades Riboccone |
| PSV001-HY033 (Valve) | AY457 | Solenoid-Outlet Bag filter auto release PDIT001>atm |
| LP003 (Valve) | AY013 | Solenoid-Outlet Bag filter vacuum breaker solenoid valve |
| LP003 (Valve) | ZSL019 | Sensor-Closed valve LP003 position confirmation |
| LP003 (Valve) | ZSH019 | Sensor-Open valve LP003 position confirmation |
| LP005 (Valve) | AY012 | Solenoid-Vacuum breaker nitrogen line |
| LP005 (Valve) | ZSL009 | Sensor-Closed valve LP005 position confirmation |
| LP005 (Valve) | ZSH009 | Sensor-Open valve LP005 position confirmation |
| LP004 (Valve) | AY011 | Solenoid-Vacuum breaker Silencer line |
| LP004 (Valve) | ZSL008 | Sensor- Closed valve LP004 position confirmation |
| LP004 (Valve) | ZSH008 | Sensor-Open valve LP004 position confirmation |
| PRV012 | ZIC001 | Proportional Valve- Outlet Bag filter Vacuum pressure Control |
| PRV012 | ZIT001 | Feedback- Outlet Bag filter Vacuum pressure Control |
| LP003 (Valve) | AY010 | Solenoid-Vacuum pressure Control nitrogen line |
| LP003 (Valve) | ZSL007 | Sensor- Closed valve LP003 position confirmation |
| LP003 (Valve) | ZSH007 | Sensor-Open valve LP003 position confirmation |
| LP002 (Valve) | AY009 | Solenoid-Vacuum pressure Control Silencer line |
| LP002 (Valve) | ZSL006 | Sensor- Closed valve LP002 position confirmation |
| LP002 (Valve) | ZSH006 | Sensor-Open valve LP002 position confirmation |
| AMOT-M05(Motor) | PG001-M05-VFD | Motor-Vacuum pump |
| AMOT-M03(Motor) | VK016-M01-VFD | Motor-Rotating drive |
| HY002 (Valve) | AY002 | Solenoid-Inject pressure to the top of VK016 |
| HY023 (Valve) | AY023 | Solenoid-Cleaning Blades Riboccone |
| HY024 (Valve) | AY024 | Solenoid-Cleaning with Gas from Below |
| HY014 (Valve) | AY014 | Solenoid-Cleaning Exhaust valve gasket |
| HY015 (Valve) | AY015 | Solenoid-Cleaning Exhaust valve gasket |
| HY008 (Valve) | AY008 | Solenoid-Air knocker ctrl sys |
| HY007A/B (Valve) | AY007A | Solenoid-Discharge valve Open position |
| HY007A/B (Valve) | AY007B | Solenoid-Discharge valve Close position |
| HY007A/B (Valve) | ZSL004 | Sensor- Closed Discharge valve position confirmation |
| HY007A/B (Valve) | ZSH004 | Sensor- Open Discharge valve position confirmation |
| VK016-PDIT001 (H) | PDIT001 | Sensor- Bag filter pressure difference transmitter |
| TIT002 (H) | TIT002 | Sensor- Process temperature sensor transmitter |
| AIT001 -AY027 (H) | AY027 | Indicator-Outlet Bag filter Control unit (Zirconia oxygen analyzer) |

## CB system bag unloading station Glove Box.

The description in the following section refers to the Glove Box, see the P&ID for actual component tag numbers.

This equipment is designed to safely and effectively manage the disposal of materials as they are introduced into the system, as well as any associated waste it produces. It is composed of 6 principal subsections.

- Prechamber. This is the point at which the operator inserts the small bags of material, and the system is triggered with actuators and sensors to move the gate into the correct position.

- Chamber. Here, the operator is able to safely handle the bag without contact with the material, and a cutter is used to open the bag and release the contents into the hopper.

- Extractor and filter. This component is designed to capture any airborne particles.

- Compactor. Once the bag is opened, the material is compressed and stored in the compactor.

- Hopper. This hopper receives the product and optimally feeds it into the transport screw.

- Transport screw and outlet valve. The product is moved through the transport screw to the pipe, and when the valve is opened, the material is released to the next equipment.

### Control device instrumentation bag unloading station Glove Box.

The following table shows the CB bag unloading station Glove Box devices.

| Device | Tag | Description |
| --- | --- | --- |
| HY035 (Valve) | AY035 | Solenoid- Bar pistons |
| HY032 (Valve) | AY032 | Solenoid- Prechamber pistons |
| HY027(Valve) | AY033 | Solenoid- Glovebox Inlet shutter |
| HY026(Valve) | AY026 | Solenoid- Glovebox Outlet shutter |
| HY031(Valve) | AY031 | Solenoid- Inflatable seal Inlet |
| PSH014 | PSH014 | Sensor-Pressure switch inflatable seal inlet |
| HY021(Valve) | AY021 | Solenoid- Inflatable seal Outlet |
| PSH015 | PSH015 | Sensor-Pressure switch inflatable seal outlet |
| HY019(Valve) | AY019 | Solenoid- Glovebox knife |
| HY018(Valve) | AY018 | Solenoid- Compactor pistons |
| GK002 | KE002-M01-VFD | Motor-Ventilation fan |
| TIT001 | TIT001 | Sensor- Process temperature sensor transmitter |
| PIT001 (H) | PIT001 | Sensor-Pressure transmitter (inside CX001) |
| GC002-PDIT003 (H) | PDIT003 | Sensor- Filter pressure difference transmitter |
| ZSL002 | ZSL002 | Sensor- Prechamber close confirmation |
| ZSH002 | ZSH002 | Sensor- Prechamber open confirmation |
| ZSL003 | ZSL003 | Sensor- Prechamber close confirmation |
| ZSH003 | ZSH003 | Sensor- Prechamber open confirmation |
| ZSL001 | ZSL001 | Sensor- Inlet shutter close confirmation |
| ZSH001 | ZSH001 | Sensor- Inlet shutter open confirmation |
| ZSL012 | ZSL012 | Sensor- Outlet shutter close confirmation |
| ZSH012 | ZSH012 | Sensor- Outlet shutter open confirmation |
| ZSL020 | ZSL020 | Sensor- Bar position close confirmation |
| ZSH020 | ZSH020 | Sensor- Bar position open confirmation |
| ZSL014 | ZSL014 | Sensor- compactor position Low Low |
| ZSHL015 | ZSHL015 | Sensor- compactor position Low |
| ZSH014 | ZSH014 | Sensor- compactor position Hi |
| ZSL016 | ZSL016 | Sensor- compactor position close confirmation |
| HY001(Valve) | AY001 | Solenoid-Fluidization |
| AMOT-M01(Motor) | NC005-M01 | Motor-Screw Conveyor |
| AMOT-M02(Motor) | NC005-M02-VFD | Motor-Screw Conveyor |
| HY004 (Valve) | AY004 | Solenoid-inject pressure to NC005 |
| LP006 (Valve) | AY025 | Solenoid-Outlet valve of the screw conveyor |
| LP006 (Valve) | ZSL017 | Sensor-Closed valve LP006 position confirmation |
| LP006 (Valve) | ZSH017 | Sensor-Open valve LP006 position confirmation |

## Magnetic filters.

The description in the following section refers to the magnetic filters, see the P&ID for actual component tag numbers.

Magnetic filters are an invaluable tool for production, as they are able to utilize a magnetic force to filter out any unwanted materials from the batch that is being processed. This ensures that only the desired components are present in the final product, allowing for a higher quality output.

### LFP-NCO system control device instrumentation CA021.

The following table shows the CA021 devices.

| Device | Tag | Description |
| --- | --- | --- |
| LP422 (Valve) | AY425 | Solenoid-Inlet valve of the filter |
| LP422 (Valve) | ZSL424 | Sensor-Closed valve LP422 position confirmation |
| LP422 (Valve) | ZSH423 | Sensor-Open valve LP422 position confirmation |
| AY446 | AY446 | Magnetic filter indicator |
| LP426 (Valve) | AY429 | Solenoid-Outlet valve of the filter |
| LP426 (Valve) | ZSL428 | Sensor-Closed valve LP426 position confirmation |
| LP426 (Valve) | ZSH427 | Sensor-Open valve LP426 position confirmation |

### CB system control device instrumentation CA011.

The following table shows the CA011 devices.

| Device | Tag | Description |
| --- | --- | --- |
| AY021 | AY021 | Magnetic filter indicator |
| LP008 (Valve) | AY022 | Solenoid-Outlet valve of the filter |
| LP008 (Valve) | ZSL011 | Sensor-Closed valve LP008position confirmation |
| LP008 (Valve) | ZSH011 | Sensor-Open valve LP008 position confirmation |
| AMOT-M01(Motor) | NO001-M01-VFD | Motor- vibrator |

# Functional description

## Mode of Operation and Machine Behavior

The project or system has a behavior based on state machines. A state machine is a control model that is defined by fixed operating states with defined requirements needed to transition from one state to another. The machine's operation can be summarized by providing the current state. State machines help operators understand what the machine is doing and what is needed in order to move into a more productive state.

Figure 1: State Machine Model

The state machine model contains a set of 17 predefined states, broken into two categories. Waiting states are stable situations for the unit/machine. The machine remains in a wait state until receiving a command to enter the next state. Waiting states include Idle, Stopped, Aborted, Suspended, Held, Completed, and Execute. Acting states are where the machine must perform some transient actions. The machine performs the functions, and then automatically moves onto the next (waiting) state when it is finished. Acting states end in “ing”, and include Resetting, Starting, Clearing, Stopping, Aborting, Suspending, Unsuspending, Holding, Unholding, and Completing. Not all states have to be used, but none can be added. Additionally, no transitions not included in the state diagram are permitted. If the machine receives a command which is invalid (such as attempting to start a machine in Execute), the command is ignored. The minimum set of states that must be implemented includes the following: Idle, Execute, Stopped, Aborted. In each state, logic can be executed until a state transition occurs.

This system is designed to coordinate and execute a variety of control states for the equipments connected to it. Through the implementation of specific commands, the system will ensure the smooth and accurate operation of the connected equipment. The following shows the states in which the equipment will be controlled according to the control model.

Cathode General Equipment (Cumulative)

- State 4 – Idle – Machine not running.

- State 6 – Execute – Machine Running.

- State 17 – Completed – Sequence completed.

- State 19 – E-Stop – Emergency Stop pressed.

Dryer

- State 4 – Idle – Machine not running.

- State 3 – Starting – Machine warm up.

- State 6 – Execute – Machine Drying.

- State 16 – Completing – Drying time almost complete.

- State 17 – Completed – Drying completed.

- State 19 – E-Stop – Emergency Stop pressed.

Cooler

- State 4 – Idle – Machine not running.

- State 3 – Starting – Machine warm up.

- State 6 – Execute – Machine Cooling.

- State 16 – Completing – Cooling time almost complete.

- State 17 – Completed – Cooling completed.

- State 19 – E-Stop – Emergency Stop pressed.

Dryer/Cooler

- State 4 – Idle – Machine not running.

- State 3 – Starting – Machine warm up.

- State 6 – Execute – Machine Cooling/Drying.

- State 16 – Completing – Cooling/Drying time almost complete.

- State 17 – Completed – Cooling/Drying completed.

- State 19 – E-Stop – Emergency Stop pressed.

## State 4. Idle - Machine not running.

In state 4 the equipment is in an inactive state, resting until it receives an instruction to start a sequence based on the desired control mode. The following sections describe the Idle state in automatic or semi-automatic mode of the equipments.

### LFP-NCO system big bag unloading station NZ001.

The following table shows the idle status of the NZ001 devices.

| Big bag unloading station NZ001 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| GK002-M01-VFD | Motor for Aspiration fan GK002 | STOP |
| AY025 | Solenoid for Vibrator solenoid valves | OFF |
| AY028 | Solenoid for The piston solenoid valve gate | OFF |
| AY097 | Solenoid for inject pressure to NZ001 | OFF |
| LP036 | Discharge Valve | CLOSED |

### LFP-NCO system Hopper TE005.

The following table shows the idle status of the TE005 devices.

| Hopper TE005 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA004 | Filter command | OFF |
| AY054 | Solenoid for hooper fluidization command | OFF |
| AY055 | Solenoid for hooper fluidization command | OFF |
| AY443 | Solenoid for discharge of hoppers | OFF |
| AY056 | Solenoid for inject pressure to NC006 | OFF |
| NC006-M01 | Motor for Screw feeder NC006 | STOP |
| NC006-M02-VFD | Motor for Screw feeder NC006 | STOP |

### LFP-NCO system pneumatic transport VZ007

The following table shows the idle status of the VZ007 devices.

| Transporter VZ007 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP059 | Inlet valve | CLOSED |
| LP066 | Vent valve | OPEN |
| PRV077 | Regulator for conveying | CLOSED |
| PRV078 | Regulator for cleaning | CLOSED |
| LP088 | Discharge Valve | CLOSED |

### LFP-NCO system diverter NZ460.

The following table shows the idle status of the NZ460 devices.

| Diverter NZ460 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| AY463 | Solenoid for diverted command | OFF |
| AY464 | Solenoid for straight command | OFF |
| AY466 | Solenoid to inflate diverter gasket | OFF |

### LFP-NCO system Hopper TE009.

The following table shows the idle status of the TE009 devices.

| Hopper TE009 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA008 | Filter command | OFF |
| AY133 | Solenoid for hooper fluidization command | OFF |
| AY135 | Solenoid for hooper fluidization command | OFF |
| LP145 | Discharge Valve | CLOSED |

### LFP-NCO system Milling.

The following table shows the idle status of the milling devices controlled by NTE.

| Milling (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| A1-1002-NORMAL | NTE SYSTEM In normal operation | ON |
| A1-1002-ReadyToCh-CP | NTE1 is ready to charge powder | OFF |
| A1-1002-SysIsChargPowder | NTE1 is charging powder | OFF |
| A1-1002-TE011-ReqPowder | NTE2 requires powder | OFF |
| To be defined | NTE2 have completed the sampling of the product | OFF |
| AY475A | Solenoid for Piston extension (sampling cylinder) | OFF |
| AY475B | Solenoid for Piston retraction (sampling cylinder) | ON |
| AY476A | Solenoid for Rotation of the sampling cylinder | ON |
| AY476B | Solenoid for Rotation of sampler in rest position | OFF |
| AY473 | Solenoid for Gasket inflation | OFF |
| AY474 | Solenoid for Gasket inflation | OFF |
| LP478 | Discharge Valve | CLOSED |
| AY148A | Solenoid for NZ216 Straight position | ON |
| AY148B | Solenoid for NZ216 Reject position | OFF |

### LFP-NCO system Hopper TE011.

The following table shows the idle status of the TE011 devices.

| Hopper TE011 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA010 | Filter command | OFF |
| AY171 | Solenoid for hooper fluidization command | OFF |
| AY172 | Solenoid for hooper fluidization command | OFF |
| LP175 | Discharge Valve | CLOSED |

### LFP-NCO system pneumatic transport VZ012

The following table shows the idle status of the VZ012 devices.

| Transporter VZ012 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP195 | Inlet valve | CLOSED |
| LP181 | Vent valve | OPEN |
| PRV197 | Regulator for conveying | CLOSED |
| PRV196 | Regulator for cleaning | CLOSED |
| LP138 | Discharge Valve | CLOSED |

### LFP-NCO system Dryer HX014.

The following tables show the idle status of the HX014 devices.

| Dryer CA013 Filter (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA008 | Filter command | OFF |
| LP236 | Inlet valve | CLOSED |

| Dryer HX014 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP239 | Inlet valve | CLOSED |
| AY333A | Solenoid for discharge valve Open position | OFF |
| AY333B | Solenoid for discharge valve Close position | ON |
| AY450 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY451 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY447 | Solenoid for cleaning Blades Riboccone | OFF |
| AY448 | Solenoid for cleaning Blades Riboccone | OFF |
| AY449 | Solenoid for cleaning with Gas from Below | OFF |
| AY335 | Solenoid for Air knocker ctrl sys | OFF |
| HX014-M01-VFD | Motor for rotating drive | STOP |
| AY259 | Solenoid for inject pressure to the screw NC015 | OFF |
| NC015-M01-VFD | Motor for screw conveyor or feeder | STOP |

| Dryer HX014 Vacuum and bag filter (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP222 | Pressure injection valve to the bag filter unit from VX006 | CLOSED |
| LP225 | Pressure injection valve to the bag filter unit from VX006 | CLOSED |
| AY457 | Solenoid for Outlet Bag filter auto release PDIT280>atm | OFF |
| LP356 | Outlet Bag filter vacuum breaker solenoid valve | CLOSED |
| LP357 | Vacuum breaker nitrogen line valve | CLOSED |
| LP352 | Vacuum breaker Silencer line | CLOSED |
| PRV346 | Outlet Bag filter Vacuum pressure Control | CLOSED |
| LP347 | Vacuum pressure Control nitrogen line | CLOSED |
| LP342 | Vacuum pressure Control Silencer line | CLOSED |
| PG398-M03 | Motor for Vacuum pump | STOP |

### LFP-NCO system Cooler VK016.

The following tables show the idle status of the VK016 devices.

| Cooler VK016 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP264 | Inlet valve | CLOSED |
| AY302A | Solenoid for discharge valve Open position | OFF |
| AY302B | Solenoid for discharge valve Close position | ON |
| AY455 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY456 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY452 | Solenoid for cleaning Blades Riboccone | OFF |
| AY453 | Solenoid for cleaning Blades Riboccone | OFF |
| AY454 | Solenoid for cleaning with Gas from Below | OFF |
| AY304 | Solenoid for Air knocker ctrl sys | OFF |
| VK016-M01-VFD | Motor for rotating drive | STOP |

| Cooler VK016 Vacuum and bag filter (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP269 | Pressure injection valve to the bag filter unit from VX008 | CLOSED |
| LP271 | Pressure injection valve to the bag filter unit from VX008 | CLOSED |
| AY242 | Solenoid for Outlet Bag filter auto release PDIT276>atm | OFF |
| LP322 | Outlet Bag filter vacuum breaker solenoid valve | CLOSED |
| LP323 | Vacuum breaker nitrogen line valve | CLOSED |
| LP318 | Vacuum breaker Silencer line | CLOSED |
| PRV313 | Outlet Bag filter Vacuum pressure Control | CLOSED |
| LP314 | Vacuum pressure Control nitrogen line | CLOSED |
| LP309 | Vacuum pressure Control Silencer line | CLOSED |
| PG400-M04 | Motor for Vacuum pump | STOP |
| LP401 | Inlet valve of the cooler Vacuum pump | CLOSED |

### LFP-NCO system pneumatic transport VZ017

The following table shows the idle status of the VZ017 devices.

| Transporter VZ017 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP363 | Inlet valve | CLOSED |
| LP370 | Vent valve | OPEN |
| LP485 | Valve equalize pressure with VK016 | OPEN |
| AY384 | Solenoid for conveying | OFF |
| AY378 | Solenoid for cleaning | OFF |

### LFP-NCO system Hopper TE019.

The following table shows the idle status of the TE019 devices.

| Hopper TE011 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA018 | Filter command | OFF |
| AY435 | Solenoid for hooper fluidization command | OFF |
| AY436 | Solenoid for hooper fluidization command | OFF |
| AY038 | Solenoid for inject pressure to NC020 | OFF |
| NC020-M01 | Motor for Screw feeder NC020 | STOP |
| NC020-M02-VFD | Motor for Screw feeder NC002 | STOP |

### LFP-NCO system magnetic filters CA021.

The following table shows the idle status of the VZ021 devices.

| Magnetic filters CA021 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP422 | Inlet valve | CLOSED |
| LP426 | Discharge Valve | CLOSED |

### CB system bag unloading station Glove Box.

The following table shows the idle status of the station Glove Box devices.

| CB bag unloading station Glove Box (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| AY035 | Solenoid for Bar pistons | OFF |
| AY032 | Solenoid for Prechamber pistons | OFF |
| AY033 | Solenoid for Glovebox Inlet shutter | ON |
| AY026 | Solenoid for Glovebox Outlet shutter | ON |
| AY031 | Solenoid for Inflatable seal Inlet | ON |
| AY021 | Solenoid for Inflatable seal Outlet | ON |
| AY019 | Solenoid for Glovebox knife | OFF |
| AY018 | Solenoid for Compactor pistons | OFF |
| KE002-M01-VFD | Motor for Ventilation fan | STOP |
| AY001 | Solenoid for Fluidization | OFF |
| NC005-M01 | Motor for Screw feeder NC005 | STOP |
| NC005-M02-VFD | Motor for Screw feeder NC005 | STOP |
| AY004 | Solenoid for inject pressure to NC005 | OFF |
| LP006 | Discharge Valve | CLOSED |

### CB system Dryer/Cooler VK006.

The following tables show the idle status of the VK006 devices.

| Dryer/cooler VK006 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP009 | Inlet valve | CLOSED |
| AY007A | Solenoid for discharge valve Open position | OFF |
| AY007B | Solenoid for discharge valve Close position | ON |
| AY014 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY015 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY028 | Solenoid for cleaning Blades Riboccone | OFF |
| AY023 | Solenoid for cleaning Blades Riboccone | OFF |
| AY024 | Solenoid for cleaning with Gas from Below | OFF |
| AY008 | Solenoid for Air knocker ctrl sys | OFF |
| VK016-M03-VFD | Motor for rotating drive | STOP |
| AY008 | Solenoid for inject pressure to the top of VK016 | OFF |

| Dryer/cooler VK006 Vacuum and bag filter (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP006 | Pressure injection valve to the bag filter unit | CLOSED |
| AY033 | Solenoid for Outlet Bag filter auto release PDIT001>atm | OFF |
| LP009 | Outlet Bag filter vacuum breaker solenoid valve | CLOSED |
| LP005 | Vacuum breaker nitrogen line valve | CLOSED |
| LP004 | Vacuum breaker Silencer line | CLOSED |
| PRV012 | Outlet Bag filter Vacuum pressure Control | CLOSED |
| LP003 | Vacuum pressure Control nitrogen line | CLOSED |
| LP002 | Vacuum pressure Control Silencer line | CLOSED |
| PG001-M05 | Motor for Vacuum pump | STOP |

### CB system pneumatic transport VZ007

The following table shows the idle status of the VZ007 devices.

| Transporter VZ017 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP001 | Inlet valve | CLOSED |
| AY005 | Solenoid for conveying | OFF |

### CB system Hopper TE009.

The following table shows the idle status of the TE009 devices.

| Hopper TE009 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA008 | Filter command | OFF |
| LP426 | Discharge Valve | CLOSED |
| NC010-M01 | Motor for rotational unloader | STOP |

### CB system magnetic filters CA011.

The following table shows the idle status of the VZ021 devices.

| Magnetic filters CA021 (Idle). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| NO001-M01-VFD | Motor for vibrator | STOP |
| LP426 | Discharge Valve | CLOSED |

## State 3. Starting - Machine warm up.

In state 3, the teams involved begin the sequence to reach the appropriate process temperatures and maintain temperature control during the process. In this section, it is necessary to refer to the diagram P&ID "217108-PID-004 " to obtain a better understanding of the process. The following sections describe the process. starting state in automatic or semiautomatic mode of the equipment.

### LFP-NCO system. Dryer HX014 Starting.

In this section, the sequence that has the dryer in the state starting is displayed.

| Permissive list |  |  |
| --- | --- | --- |
| The system must be running. No emergency alarms present. |  |  |
| Step | Action | Completion criteria |
| 1 | The dryer should receive the Start command | System is running |
| 2 | Minimum Temperature Setpoint “SP_MinTemperature” greater than the temperature of the TT026 transmitter to start the temperature pump (PA005-M005-VFD). TT026> SP_MinTemperature Start PA-005-M005-VFD | PA005-M005-VFD Running |
| 3 | Tank outlet temperature (TT013) greater than or equal to the process temperature Setpoint “SP_ProsTemperature” TT013>= SP_ProsTemperature | TT013>= SP_ProsTemperature |
| 4 | Open the LC005 (AY005) valve and run the Dryer process pump (PA008-M008-VFD). Start PA008-M008-VFD + Open LC005 (AY005) | PA008-M008-VFD Running |
| 5 | Starts timer to reach temperature at dryer inlet “T_IncreaseTemperature” | Run Timer “T_IncreaseTemperature” |
| 6 | The oil inlet temperature of the dryer (TT021) is higher than the set inlet oil temperature “SET_InletOilTemperature” Oil Inlet Temperature > “SET_InletOilTemperature” Reset timer “T_IncreaseTemperature” Start temperature control |  |
| 7 | The system controls the temperature by opening and closing the LC005 (AY005) valve. |  |
| 8 | Dryer started. |  |

### LFP-NCO system. Cooler VK016 Starting.

In this section, the sequence that has the Cooler in the state starting is displayed.

| Permissive list |  |  |
| --- | --- | --- |
| The system must be running. No emergency alarms present. |  |  |
| Step | Action | Completion criteria |
| 1 | The Cooler should receive the Start command | System is running |
| 2 | Run Water condensed chilling unit GB003. |  |
| 3 | Start inlet Pump Water condensed chilling unit GB003. Run PA001-M001 | PA001-M001 Running |
| 4 | Start outlet Pump TA001. Run PA001-M002 | PA001-M002 Running |
| 5 | Start inlet Pump HE001. Run PA001-M003 | PA001-M003 Running |
| 6 | Start temperature control in HVAC chilled ring with valve LC001 (AY001) |  |
| 7 | Start temperature control on HE001 and pump PA001-M003 |  |
| 8 | Run PA004-M004-VFD | PA004-M004-VFD Running |
| 9 | When the oil reaches the process temperature TT013<= SP_ProsTemperature | TT013<= SP_ProsTemperature |
| 10 | The LC002 (AY002) valve opens, and the Cooler process pump (PA006-M006-VFD) starts. | PA006-M006-VFD Running |
| 11 | Starts timer to reach temperature at cooler inlet “T_DecreaseTemperature” | Run Timer “T_DecreaseTemperature” |
| 12 | The oil inlet temperature of the cooler (TT022) is lower than the set inlet oil temperature “SET_InletOilTemperature” Oil Inlet Temperature < “SET_InletOilTemperature” Reset timer “T_DecreaseTemperature” Start temperature control |  |
| 13 | The system controls the temperature by opening and closing the LC002 (AY002) valve. |  |
| 14 | Cooler started. |  |

### CB system. Dryer/Cooler VK006 Starting.

In this section, the sequence that has the Dryer/Cooler in the state starting is displayed.

| Permissive list |  |  |
| --- | --- | --- |
| The system must be running. No emergency alarms present. |  |  |
| Step | Action | Completion criteria |
| 1 | The Dryer/Cooler should receive the Start command | System is running |
| 2 | Steps 2 and 3 of the Dryer starting sequence must be fulfilled. |  |
| 3 | Step 2 through Step 9 of the Cooler starting sequence must be fulfilled. |  |
| 4 | Run the Dryer/Cooler process pump (PA007-M007-VFD) in recirculation mode. Start PA007-M007-VFD | PA007-M007-VFD Running |
|  | Dryer/Cooler started. |  |

## State 6. Execute - Machine Running.

In state 6 the equipment is in sequence execution or processing activity in the mode selected by the user. The following sections describe the Execute state in automatic or semi-automatic mode of the equipment.

### LFP-NCO system. Unloading of big bags in automatic sequence from NZ001 to TE005.

The sequence can be started by the local HMI or from the MES remotely. It will firstly consist on a scan of the big bag that will be dumped in order to give to the MES the data of the big bag. If the MES gives the green light, the operator can start the sequence of dumping of the LFP Bag.

| Permissive list |  |  |
| --- | --- | --- |
| Barcode scan performed & permissive to dump from the MES System Big bag positioned on M501 valve. No high level on TE005 (LSH039 not active) No emergency alarms present |  |  |
| Step | Action | Completion criteria |
| 1 | Aspiration fan GK002 | GK002 Running |
| 2 | Filter CA004 starts | CA004 Running AND Pl_piston Active |
| 3 | The piston solenoid valves (AY028) are energized. | HY028 ZSH Active AND Pl_piston Active |
| 4 | The piston solenoid valves (AY028) are de-energized. | HY028 ZSL Active AND pedale_guarnizione Active |
| 5 | The inflation seal valves (AY025) are energized. | HY025 ZSH Active AND pl_start_unload Active |
| 6 | The inlet valve of the hopper opens (LP036) | Inlet valve opened (ZSH038 ACTIVE) |
| 7 | Loading. | High level of the hopper (LSH039 ACTIVE) OR T_loadTE005 elapsed OR pl_start_unload Active |
| 8 | The inlet valve of the hopper closes (LP036) | Inlet valve closed (ZSL037 ACTIVE) AND pedale_guarnizione Active |
| 9 | The inflation seal valves (AY025) are de-energized. | HY025 ZSL Active |
| 10 | If the timer “T_loadTE005” elapse, or the high level is activated the system will close the hopper inlet valve LP036, timer “T_RunGK002” starts and timer “T_RunCA004” starts | ZSL037 Active |
| 11 | If the timer “T_RunGK002” elapse the system will Stop GK002. | GK002 Stop |
| 12 | If the timer “T_RunCA004” elapse the system will Stop CA004. | CA004 Stop |

### LFP-NCO system. Unloading from TE005 to VZ007

The next step is transporting the product stored into TE005 to the transporter and convey it to the hopper TE009, which is the buffer hopper before the milling system (not supplied by NTE) or the hopper TE011 which is the buffer hopper before the dryer system.

In automatic mode, the operator will firstly declare the quantity of product that will be conveyed (expressed in kilos) in the HMI, and then the system will start accordingly. The screw feeder dosing will be operative once the kilos transported are nearby to the limit.

The destination will change accordingly to the product selected. In case of LFP, the destination will be TE009. In case of NMC, the destination will be TE011.

The first step is to unload the product to the transporter VZ007.

| Permissive list |  |  |
| --- | --- | --- |
| No High level on VZ007 (LSH070 not active). The transporter VZ007 must be available and not running. No emergency alarms present. |  |  |
| Step | Action | Completion criteria |
| 1 | Transporter vent valve LP066 Opens | ZSL067 active |
| 2 | Transporter inlet valve LP059 Opens | ZSH060 active |
| 3 | Run Filter CA004 | CA004 Running |
| 4 | Fluidization cycle start (AY054 and AY055) | Fluidization cycle Running |
| 5 | The TE005 discharge valve AY443 opens |  |
| 6 | The screw feeder NC006 starts at a certain speed (to be defined during commissioning) | NC006-M02-VFD Running |
| 7 | Bridge Breaker NC006 starts | NC006-M01 Running |
| 8 | A timer “T_loadVZ007” starts |  |
| 9 | Loading. | High level of the transporter (LSH070 ACTIVE) or WT087 = weight target or T_loadVZ007 elapsed or LS092 ACTIVE |
| 10 | Once loading is completed, by the activation or by the elapsing of the timer, the bridge breaker and the screw feeder will be deactivated | NC006-M01 Stopped NC006-M02-VFD Stopped |
| 11 | The discharge valve AY443 closes |  |
| 12 | The Inlet valve LP059 Closes | ZSL061 Active |
| 13 | Fluidization cycle stops (AY054 and AY055) and timer “T_RunCA004” starts | Fluidization cycle stops |
| 14 | If the timer “T_RunCA004” elapse the system will Stop CA004. | CA004 Stop |
| 15 | If the timer “T_loadTE005” elapse, or the high level is activated the system will close the hopper inlet valve LP036 | ZSL036 Active |

#### LFP-NCO System.

### LFP-NCO system. Conveying from VZ007 to TE009 (LFP mode selected)

Once loading is completed, the Transporter will firstly pressurize and then convey the product to the destination hopper.

| Permissive list |  |  |
| --- | --- | --- |
| No emergency alarms present. Loading Phase completed. No High level in TE009. Deviator Valve NZ460 in straight position (ZSL461 Active). |  |  |
| Step | Action | Completion criteria |
| 1 | Transporter vent valve LP066 closes | ZSL068 active |
| 2 | Diverter in Position and Inflated | (NZ460_Straight Active OR NZ460_Diverted Active) AND NZ460_Inflated |
| 3 | Run Filter CA008 | CA008 Running |
| 4 | The transporter discharge valve LP088 opens. | ZSH089 Active |
| 5 | The PRV077 opens at a certain ratio to achieve the pressure set | ZIC077 = TransportPressure ZIT07 = TransportPressure |
| 6 | The transporter is now pressurizing. A timer “T_PressurizationVZ007” will start |  |
| 7 | Pressure sensed at transporter PIT075 > H level | PIT075 > H level |
| 8 | The transporter is now in convey phase. A timer “T_conveyVZ007” start | Timer elapsed or PIT < L level |
| 9 | The PRV077 closes at 0% |  |
| 10 | The Discharge valve LP088 closes | ZSL090 Active |
| 11 | A timer “T_depressurizationVZ007” starts | PIT075 < LL |
| 12 | The Vent valve LP066 opens and timer “T_RunCA008” starts | ZSH067 Active |
| 13 | Once PIT075 < L level the system goes in standby | PIT075 < LL level |
| 14 | System is in standby, if another cycle is required the transporter will carry on the loading phase and consequently the convey phase. If no more cycles are required the transporter will remain in standby mode. |  |
| 15 | If the timer “T_RunCA008” elapse the system will stop CA008. | CA008 Stop |

### LFP-NCO system. Conveying from VZ007 to TE011 (NMC Mode selected)

| Permissive list |  |  |
| --- | --- | --- |
| No emergency alarms present. Loading Phase completed. No High level in TE011. Deviator Valve NZ460 in diverted position (ZSH462 Active). |  |  |
| Step | Action | Completion criteria |
| 1 | Transporter vent valve LP066 closes | ZSL068 active |
| 2 | Diverter in Position and Inflated | (NZ460_Straight Active OR NZ460_Diverted Active) AND NZ460_Inflated |
| 3 | Run Filter CA008 | CA008 Running |
| 4 | The transporter discharge valve LP088 opens. | ZSH089 Active |
| 5 | The PRV077 opens at a certain ratio to achieve the pressure set | ZIC077 = TransportPressure ZIT07 = TransportPressure |
| 6 | The transporter is now pressurizing. A timer “T_PressurizationVZ007” will start |  |
| 7 | Pressure sensed at transporter PIT075 > H level | PIT075 > H level |
| 8 | The transporter is now in convey phase. A timer “T_conveyVZ007” start | Timer elapsed or PIT < L level |
| 9 | The PRV077 closes at 0% |  |
| 10 | The Discharge valve LP088 closes | ZSL090 Active |
| 11 | A timer “T_depressurizationVZ007” starts | PIT075 < LL |
| 12 | The Vent valve LP066 opens and timer “T_RunCA008” starts | ZSH067 Active |
| 13 | Once PIT075 < L level the system goes in standby | PIT075 < LL level |
| 14 | System is in standby, if another cycle is required the transporter will carry on the loading phase and consequently the convey phase. If no more cycles are required the transporter will remain in standby mode. |  |
| 15 | If the timer “T_RunCA008” elapse the system will stop CA008. | CA008 Stop |

### LFP-NCO system. Unloading of LFP from TE009 to Milling System

In this phase, the product stored into TE009 will be discharged in order to be processed by the milling system (Not in NTE scope).

The sequences for the unloading from TE009 to milling system is the following:

| Permissive list |  |  |
| --- | --- | --- |
| No emergency alarms present. NTE system ready to charge powder. Milling system in normal operation. |  |  |
| Step | Action | Completion criteria |
| 1 | Milling system send the signal “CP Requires powder” to NTE System | Milling system send the signal “CP Requires powder” to NTE System |
| 2 | Run Filter CA008 | CA008 Running |
| 3 | Fluidization cycle start (AY133 and AY135) | Fluidization cycle Running |
| 4 | Milling system open the Load valve |  |
| 5 | TE009 discharge valve LP145 opens | ZSH147 Active |
| 6 | Powder charging in progress. A timer “T_MillCharge” will start | LS137 is active or Timer “T_MillCharge” elapsed |
| 7 | TE009 discharge valve closes | ZSL146 Active |
| 8 | Fluidization cycle stops (AY133 and AY135) and timer “T_RunCA008” starts | Fluidization cycle stops |
| 9 | Milling system closes the load valve |  |
| 10 | If the timer “T_RunCA008” elapse the system will Stop CA008. | CA008 Stop |
| 11 | Loading of milling system completed |  |

Once milling process has been completed, the mill system will send a contact to NTE Plc that is available to discharge the powder. Prior to the full discharge it is possible to perform a sampling of the product in order to check whether the quality is suitable or not.

### LFP-NCO system. Sampling of the product after milling phase

Once milling phase is completed, it can be launched the sampling sequence from the HMI. It mainly consists on a small extraction of the product into the sampler cylinder. Once extracted, the powder will be sent to QC laboratory in order to be analyzed.

| Permissive list |  |  |
| --- | --- | --- |
| No emergency alarms present. Milling system in normal operation. Milling system ready to discharge powder. Sampler available. |  |  |
| Step | Action | Completion criteria |
| 1 | Operator launch the sampling command by the HMI |  |
| 2 | A timer “T_sample” will start |  |
| 3 | Activation of solenoid valve AY475A for piston extension | ZSH480 Active |
| 4 | Rotation of the sampling cylinder by the activation of AY476A |  |
| 5 | Gasket inflation trough solenoid valve AY474 and 473 |  |
| 6 | Powder starts to discharge from Mill |  |
| 7 | Timer “T_sample” elapse |  |
| 8 | Gasket deflation trough deactivation of solenoid valve AY474 and 473 |  |
| 9 | Activation of solenoid valve AY475B for piston retraction | ZSL479 Active |
| 10 | Rotation of sampler in rest position in order to get the sample powder |  |
| 11 | Sampling sequence completed |  |

### LFP-NCO system. Unloading from mill to TE011

Assuming that the powder sample is okay and approved by the Quality, the next step is to load the TE011 with the powder of the mill. Here below the sequence of steps for the TE011 loading.

Note: if the product quality is good the system has to ensure that the deviator valve is in correct position.

| Permissive list |  |  |
| --- | --- | --- |
| No emergency alarms present. Milling system in normal operation. Milling system ready to discharge powder. No high level in TE011 (LSH161 not active). Deviator NZ216 in straight position (ZSL150 Active). |  |  |
| Step | Action | Completion criteria |
| 1 | Mill system send a contact to NTE PLC “CP ready to discharge powder” |  |
| 2 | Run Filter CA010 | CA010Running |
| 3 | NTE PLC will send a contact to Mill “NTE_Request Powder” |  |
| 4 | Opening of the valve LP478 | ZSH481 Active |
| 5 | A timer “T_LoadTE011” starts |  |
| 6 | Powder starts to discharge from Mill | High level of the hopper (LS161 ACTIVE) OR T_loadTE011 elapsed |
| 7 | Timer “T_sample” elapse |  |
| 8 | Mill system close the discharge valve |  |
| 9 | Valve LP478 closes and timer “T_RunCA010” starts | ZSL481 Active |
|  | If the timer “T_RunCA010” elapse the system will stop CA010. | CA010 Stop |
| 10 | Load completed |  |

### LFP-NCO system. Unloading from TE011 to VZ012

To transport the product stored in TE011 to the Dryer, the transporter must first be loaded. Below is the sequence to load the conveyor.

| Permissive list |  |  |
| --- | --- | --- |
| No High level on VZ012 (LSH070 not active) VZ012 must be available and not running. No emergency alarms present |  |  |
| Step | Action | Completion criteria |
| 1 | Transporter vent valve LP181 Opens | ZSH203 active |
| 2 | Transporter inlet valve LP195 Opens | ZSH204 active |
| 3 | Run Filter CA010 | CA010 Running |
| 4 | The TE011 discharge valve LP175 opens | ZSH176 Active |
| 5 | Fluidization cycle start (AY171 and AY172) | Fluidization cycle Running |
| 6 | A timer “T_loadVZ012” starts |  |
| 7 | Loading. | High level of the transporter (LSH183 ACTIVE) OR T_loadVZ012 elapsed OR LS218 ACTIVE |
| 8 | loading is completed, by the level activation or by the elapsing of the timer |  |
| 9 | The discharge valve LP175 closes | ZSL177 Active |
| 10 | The Inlet valve LP195 Closes | ZSL205 Active |
| 11 | Fluidization cycle stops (AY171 and AY172) and timer “T_RunCA010” starts | Fluidization cycle stops |
| 12 | If the timer “T_RunCA010” elapse the system will Stop CA010. | CA010 Stop |
| 13 | Loading completed |  |

### LFP-NCO system. Convey from VZ012 to Dryer HX014

Once loading is completed, the Transporter will firstly pressurize and then convey the product to the destination. This sequence includes the charging stages of the dryer.

| Permissive list |  |  |
| --- | --- | --- |
| No emergency alarms present. Loading Phase completed. Heater not in alarm. Oil Inlet Temperature > SET_InletOilTemperature |  |  |
| Step | Action | Completion criteria |
| 1 | Automatic Cycle Start Command | Oil Inlet Temperature > SET_InletOilTemperature |
| 2 | Inspection Hatch Closed Verification | ZSL262.HX014 active |
| 3 | Calculation of Temperature Control Values During Load (Dryer HX014) | T_Min_Loading = Oil Inlet Temperature - SET_MaxDifferenceDuringLoad T_Max_Loading = T_Min_Loading + SET_HysteresisDifferenceDuringLoad |
| 4 | Transporter vent valve LP181 closes | ZSL204 active |
|  | Dryer system open the Load valve LP239 | ZSH240 Active |
|  | The filter CA013 discharge valve opens LP236 | ZSH237 Active |
| 5 | Filter CA013 | CA013 Running |
| 6 | Engine Startup Riboccone CMDHX014.M01.VFD ON (Dryer HX014) | RUNHX014.M01.VFD active |
| 7 | Temperature Control Activation During Load (Dryer HX014) | DryerFeedSystem active |
| 8 | The PRV197 opens at a certain ratio to achieve the pressure set | ZIC197 = TransportPressure ZIT197 = TransportPressure |
| 9 | The transporter is now pressurizing. A timer “T_PressurizationVZ012” will start |  |
| 10 | Pressure sensed at transporter PIT194 > H level | PIT194 > H level |
|  | The transporter discharge valve opens LP138 | ZSH190 Active |
|  | The transporter is now in convey phase. A timer “T_conveyVZ012” start | Timer elapsed PIT < H level WT210 = 0 |
| 11 | The PRV197 closes at 100% |  |
| 12 | Load Terminated | Load_Finished |
| 13 | Stop Temperature Control During Loading (Dryer HX014) | DryerFeedSystem not active |
| 14 | The Discharge valve LP088 closes | ZSL190 Active |
| 15 | The filter CA013 discharge valve LP236 closes | ZSL238 Active |
| 16 | Dryer system closes the Load valve LP239 | ZSL241 Active |
| 17 | A timer “T_depressurizationVZ012” starts |  |
| 18 | The Vent valve LP181 opens and timer “T_RunCA013” starts | ZSH203 Active |
|  | Once PIT194 < L level the system goes in standby | PIT194 < L level |
|  | System is in standby, if another cycle is required the transporter will carry on the loading phase and consequently the convey phase. If no more cycles are required the transporter will remain in standby mode. |  |
|  | If the timer “T_RunCA013” elapse the system will stop CA013. | CA013 Stop |

### LFP-NCO system. Drying sequence Dryer HX014

#### LFP-NCO Drying Sequence Without Bypass

The following sequence shows the main actions that the dryer performs during its process, when the bypass is turned off

| Permissive list |  |  |
| --- | --- | --- |
| *Add alarms that you consider essential OKAWARA has not considered any* Automatic mode Oil Inlet Temperature > SET_InletOilTemperature |  |  |
| Step | Action | Completion criterion |
| 1 | Vacuum Pump Motor Start CMDPG398.M01.VFD ON + LP356(AY362.HX014) vacuum breaker solenoid valve opening control+ Waiting Time Vacuum Pump Startup | RUNPG398.M01.VFD active + Elapsed Vacuum Pump Start Time DLYT_VacuumPump_SwitchOn |
| 2 | LP356(AY362.HX014) Vacuum Breaker Solenoid Valve Closure |  |
| 3 | Vacuum Control Activation PRV346 valve controlled by differential pressure PDIT280 loop. | VacuumControlActive ON |
| 4 | Reaching Internal Temperature Threshold | Indoor Temperature > SP_Production_Temperature |
| 5 | Waiting Time Execution Process | Elapsed Process Execution Time DLYT_Production_Duration |
| 6 | Vacuum Pump Motor Stop CMDPG398.M01.VFD OFF + Stop Vacuum Control | RUNPG398.M01.VFD not active + VacuumControlActive not active |
| 7 | Filter Cleaning Sequence Activation Waiting time Filter cleaning | Elapsed Filter Cleaning Time DLYT_Cleaning_Filter |
| 8 | Stop Filter Cleaning Sequence |  |
| 9 | Solenoid valve opening control Cleaning with Gas from Below AY449.HX014 ON | Internal Pressure > TH_ProductionVacuum |
| 10 | Solenoid valve closing command Cleaning with Gas from Below AY449.HX014 OFF |  |
| 11 | Stop Motor Rebound CMDHX014.M01.VFD OFF | RUNHX014.M01.VFD inactive |
| 12 | Sending Signal For Discharge Enablement Request | Discharge_ON active |

#### LFP-NCO Drying Sequence With Bypass

In case of an issue or error with the dryer system, a bypass mode has been created. This is selectable from the HMI, and when enabled, the dryer will act as a conveying system only, passing the material directly through to the Cooling System

### LFP-NCO system. Unloading from Dryer HX014 to Cooler VK016

| Necessary conditions |  |  |
| --- | --- | --- |
| *Add alarms that you consider essential OKAWARA has not considered any* Automatic mode Oil Inlet Temperature < SET_InletOilTemperature |  |  |
| Step | Action | Completion criterion |
| 1 | Automatic Mode Active + Automatic Cycle Start Command | Oil Inlet Temperature < SET_InletOilTemperature |
| 2 | Inspection Hatch Closed Verification | ZSL297.VK016 active |
| 3 | Calculation of Temperature Control Values During Load | T_Max_Loading = Oil Inlet Temperature + SET_MaxDifferenceDuringLoad T_Min_Loading = T_Max_Loading - SET_HysteresisDifferenceDuringLoad |
| 4 | Load solenoid valve opening control LP264(AY329.VK016) | ZSL265.VK016 not active + ZSH266.VK016 active |
| 5 | Engine Startup Riboccone CMDVK016.M01.VFD ON | RUN VK016.M01.VFD active |
| 6 | Temperature Control Activation During Load | Active CoolerFeedSystem |
| 7 | Sending Signal For Charge Enablement Request (VK016). |  |
| 8 | Receiving Valley Station Signal of Discharge Enablement (HX014). | Discharge_Release active (HX014). |
| 9 | The screw feeder NC015 starts at a certain speed + AY259.HX014 ON | NC015-M01-VFD Running |
| 10 | HY333A/B.HX014 exhaust solenoid valve opening control AY333A.HX014 ON AY333B.HX014 OFF + AirKnocker Sequence Activation (AY335.HX014) | ZSH263.HX014 not active + AirKnocker_ON active |
| 11 | Waiting time Discharge | Elapsed Discharge Time DLYT_Discharge |
| 12 | Solenoid valve opening command for cleaning Blades Riboccone AY447.HX014 ON + AY448.HX014 ON + Engine Startup Riboccone CMDHX014.M01.VFD ON | RUNHX014.M01.VFD active |
| 13 | Waiting Time Cleaning Blades Riboccone | Time Cleaning Ribbon Blades spent DLYT_CleaningRibbonBlade |
| 14 | Stop Engine Rebound CMDHX014.M01.VFD OFF + AirKnocker Sequence Stop(AY335.HX014) + Command opening solenoid valve cleaning gasket drain valve AY450.HX014 ON + AY451.HX014 ON Electrovalve Cleaning Blades Riboccone closing command AY447.HX014 OFF + AY448.HX014 OFF | RUNHX014.M01.VFD inactive + AirKnocker_ON not active |
| 15 | Waiting Time Cleaning Exhaust Valve Gasket | Time spent cleaning Exhaust Valve Gasket DLYT_CleaningDischargeSeal |
| 16 | HY333A/B.HX014 exhaust solenoid valve closing control AY333A.HX014 OFF AY333B.HX014 ON + Stop Signal For Discharge Enablement Request | ZSH263.HX014 active + Discharge_ON not active |
| 17 | Screw feeder NC015 will be deactivated+ AY259.HX014 OFF | NC015-M01-VFD Stopped |
| 18 | Receiving Signal from HX014 Station of Load Terminated | Load_Finished |
| 19 | Temperature Control Stop During Loading | CoolerFeedSystem not active |
| 20 | LP264 load solenoid valve closing control AY329.VK016 OFF | ZSL265.VK016 active + ZSH266.VK016 inactive |

### LFP-NCO system. Cooler sequence Cooler VK016

#### LFP-NCO Cooler Sequence without bypass

The following sequence shows the main actions that the cooler performs during its process.

| Necessary conditions |  |  |
| --- | --- | --- |
| *Add alarms that you consider essential OKAWARA has not considered any* Automatic mode Oil Inlet Temperature < SET_InletOilTemperature |  |  |
| Step | Action | Completion criterion |
| 1 | Vacuum Pump Motor Start CMDPG400.M01.VFD ON + Gas pump solenoid valve opening control HY519.VK016 AY512.VK016 ON + Vacuum Breaker Solenoid Valve Opening Command LP322 (AY328.VK016 ) Activating Water Pump Control + Waiting Time Vacuum Pump Startup | RUN PG400.M01.VFD active + Active WaterPumpControl Elapsed Vacuum Pump Start Time DLYT_VacuumPump_SwitchOn |
| 2 | vacuum breaker solenoid valve closure LP322 (AY328.VK016 ) |  |
| 3 | Vacuum Control Activation PRV313 valve controlled by differential pressure PDIT276 loop. | VacuumControlActive ON |
| 4 | Reaching Internal Temperature Threshold | Indoor Temperature < SP_Production_Temperature |
| 5 | Waiting Time Execution Process | Elapsed Process Execution Time DLYT_Production_Duration |
| 6 | Vacuum Pump Motor Stop CMDPG400.M01.VFD OFF + Gas Pump Solenoid Valve Closure Control HY512.VK016 AY512.VK016 OFF + Stop Vacuum Control + Stop Water Pump Control | RUNPG400.M01.VFD not active + VacuumControlActive not active + WaterPumpControl not active |
| 7 | Filter Cleaning Sequence Activation Waiting time Filter cleaning | Elapsed Filter Cleaning Time DLYT_Cleaning_Filter |
| 8 | Stop Filter Cleaning Sequence |  |
| 9 | Electrovalve opening command Cleaning with Gas from Below AY454.VK016 ON | Internal Pressure > TH_ProductionVacuum |
| 10 | Solenoid valve closing command Cleaning with Gas from Below AY454.VK016 OFF |  |
| 11 | Sending Signal For Discharge Enablement Request | Discharge_ON active |

#### LFP-NCO Cooler system with bypass

In case of an issue or error with the cooler system, a bypass mode has been created. This is selectable from the HMI, and when enabled, the cooler will act as a conveying system only, passing the material directly through to the transporter VZ017

### LFP-NCO system. Unloading from Cooler VK016 to VZ017

To transport the product processed in VK016 to the hopper TE019, the conveyor VZ017 must first be loaded. Below is the sequence for loading the conveyor.

| Permissive list |  |  |
| --- | --- | --- |
| No High level on VZ017 (LSH372 not active) VZ017 must be available and not running. No emergency alarms present |  |  |
| Step | Action | Completion criteria |
| 1 | Transporter vent valve LP370 Opens | ZSH458 active |
| 2 | Transporter vent valve LP485 Opens | ZSH486 active |
| 3 | Sending Signal for Charge Enablement Request (VZ017). |  |
| 4 | Transporter inlet valve LP363 Opens | ZSH364 active |
| 5 | Receiving Valley Station Signal of Discharge Enablement. | Discharge_Release active |
| 6 | Stop Motor Rebound CMDVK016.M01.VFD OFF | RUNVK016.M01.VFD inactive |
| 7 | HY302A/B.VK016 exhaust solenoid valve opening control XY320A.VK016 ON XY302B.VK016 OFF + AirKnocker Sequence Activation | ZSH298.VK016 not active + AirKnocker_ON active |
| 8 | Waiting time Discharge | Elapsed Discharge Time DLYT_Discharge |
| 9 | Electrovalve opening command for cleaning recoil blades AY452.VK016 ON + AY453.VK016 ON + Engine Startup Riboccone CMDVK016.M01.VFD ON | RUNVK016.M01.VFD active |
| 10 | Waiting Time Cleaning Blades Riboccone | Time Cleaning Ribbon Blades spent DLYT_CleaningRibbonBlade |
| 11 | Stop Motor Rebound CMDVK016.M01.VFD OFF + AirKnocker Sequence Stop (AY304.VK016) + Solenoid valve cleaning opening control Exhaust Valve Gasket AY455.VK016 ON AY456.VK016 ON Electrovalve Cleaning Blades Riboccone closing command AY452.VK016 OFF + AY453.VK016 OFF | RUNVK016.M01.VFD inactive + AirKnocker_ON not active |
| 12 | Waiting Time Cleaning Exhaust Valve Gasket | Time spent cleaning Exhaust Valve Gasket DLYT_CleaningDischargeSeal |
| 13 | A timer “T_loadVZ017” starts |  |
| 14 | Loading. | High level of the transporter (LSH372 ACTIVE) OR T_loadVZ017 elapsed |
| 15 | loading is completed, by the level activation or by the elapsing of the timer |  |
| 16 | HY302A/B.VK016 exhaust solenoid valve closing control AY302A.VK016 OFF AY302B.VK016 ON + Stop Signal For Discharge Enablement Request | ZSH263.VK016 active + Discharge_ON not active |
| 17 | The discharge valve LP363 closes | ZSL365 Active |
| 18 | The Inlet valve LP370 Closes | ZSL459 Active |
| 19 | The Inlet valve LP485 Closes | ZSL487 Active |
| 20 | Loading completed |  |

### LFP-NCO system. Conveying from VZ017 to TE019

Once loading is completed, the Transporter will firstly pressurize and then convey the product to the destination hopper.

| Permissive list |  |  |
| --- | --- | --- |
| No emergency alarms present. Loading Phase completed. No High level in TE019 (). |  |  |
| Step | Action | Completion criteria |
| 1 | Transporter vent valve LP370 closes | ZSL459 active |
| 2 | The Inlet valve LP485 Closes | ZSL487 Active |
| 3 | Solenoid AY384 ON to achieve the pressure set and start conveying |  |
| 4 | The transporter is now pressurizing. A timer “T_PressurizationVZ017” will start |  |
| 5 | Pressure sensed at transporter PIT383 > H level | PIT383 > H level |
| 6 | Run Filter CA018 | CA018 Running |
| 7 | The transporter is now in convey phase. A timer “T_conveyVZ017” start | Timer elapsed or PIT < H level |
| 8 | The solenoid AY384 OFF |  |
| 10 | A timer “T_depressurizationVZ007” starts |  |
| 11 | The Vent valve LP370 opens | ZSH458 active |
| 12 | Transporter vent valve LP485 opens and timer “T_RunCA018” starts | ZSH486 active |
| 13 | Once PIT383 < L level the system goes in standby | PIT383 < L level |
|  | System is in standby, if another cycle is required the transporter will carry on the loading phase and consequently the convey phase. If no more cycles are required the transporter will remain in standby mode. |  |
|  | If the timer “T_RunCA018” elapse the system will stop CA018. | CA018 Stop |

### LFP-NCO system. Cleaning of VZ017

In automatic mode, the cleaning sequence is activated once the automatic cycle is stopped and if enabled into the HMI.

| Permissive list |  |  |
| --- | --- | --- |
| No emergency alarms VZ017 must be available and not running (only in semiautomatic mode) LSH415 not active Cleaning phase enabled (only in automatic mode) |  |  |
| Step | Action | Completion criteria |
| 1 | In semiautomatic mode: system perform the cleaning steps from 1 to 3 In automatic mode: system is in phase 3 | PIT383 < Low level |
| 2 | A “Timer_Empty_Line” is started | PIT383 < Low level “TR_Low pressure” |
| 3 | The cleaning solenoid (AY378) is opened A “Timer_Cleaning_Line” is started | Timer_Cleaning_Line elapsed |
| 4 | Conveying solenoid AY384 close Cleaning solenoid AY378 close |  |
| 5 | The vent valve LP370 open A “Timer_Depressurization_Too_Long” is started | ZSH458 active & PIT383 < Low level” |

### LFP-NCO system. Unloading from T019 to Slurry Preparation.

Finally, the processed product is stored in this hopper before being unloaded via an established sequence to the slurry preparation system (not in NTE scope). The slurry system will send a hardwired contact to the NTE PLC in order to allow the discharge. The details of the unloading process can be found in the accompanying table.

| Permissive list |  |  |
| --- | --- | --- |
| The magnetic filter C021 must be available. No emergency alarms present. Slurry system available and ready to receive powder |  |  |
| Step | Action | Completion criteria |
| 1 | Slurry system send a contact to start the unload sequence |  |
| 2 | Run Filter CA018 | CA018 Running |
| 3 | The C021 discharge valve LP426 opens | ZSH427 active |
| 4 | The C021 charge valve LP422 opens | ZSH423 active |
| 5 | Fluidization cycle start (AY435 and AY436) | Fluidization cycle Running |
| 6 | The screw feeder NC020 starts at a certain speed (to be defined during commissioning) | NC020-M02-VFD Running |
| 7 | Bridge Breaker NC020 starts | NC020-M01 Running |
| 8 | A timer “T_UnloadTE019” starts |  |
| 9 | Unloading. | WT421 = 0 or T_UnloadTE019 or LS421 ACTIVE |
| 10 | Once unloading is completed, by the activation or by the elapsing of the timer, the bridge breaker and the screw feeder will be deactivated | NC020-M01 Stopped NC020-M02-VFD Stopped |
| 11 | The C021 charge valve LP422 Closes | ZSL424 active |
| 12 | The C021 discharge valve LP426 Closes | ZSL428 active |
| 13 | Fluidization cycle stops (AY435 and AY436) and timer “T_RunCA018” starts | Fluidization cycle stops |
|  | If the timer “T_RunCA018” elapse the system will Stop CA018. | CA018 Stop |

### CB system, bag unloading station Glove Box.

The sequence can be started by the local HMI. It will firstly consist on a scan of the bag that will be dumped in order to give to the MES the data of the bag. If the MES gives the green light, the operator can start the sequence of dumping of the CB Bag.

The carbon black bag will be firstly inserted into the CX001 handling station in order to be cutted and to unload the carbon black. The empty bag will be disposed by the compactor CX004 that will contain all the empty bags. The following table shows the sequence for entering material into the system.

| Permissive list |  |  |
| --- | --- | --- |
| Barcode scan performed & permissive to dump from the MES System No high level on Hopper (LSH001 not active) No emergency alarms present |  |  |
| Step | Action | Completion criteria |
| 1 | The operator must verify that the product entering the system matches the expected product indicated by the scanner. Upon opening the front gate of the prechamber (Solenoid AY032 ON command from the HMI), the system will detect and register the gate open status. | ZSH002 active ZSH003 active |
| 2 | The operator must insert the bag or product into the prechamber and securely shut the gate (Solenoid AY032 OFF command from the HMI). Once the gate is closed, the system will detect the closure. | ZSL002 active ZSL003 active |
| 3 | The operator must give the instruction to activate the cut off glove box inlet gate (Solenoid AY033 ON command from the HMI). Once the gate is opened, the operator must insert their hands into the prechamber's gloves in order to facilitate the pushing of the bag through the connecting tunnel. | ZSH001 active |
| 4 | To cut the bag and access its content, the operator must raise the security bar with the command from the HMI (Solenoid AY035 ON), indicating to the system that the bar has been raised. Then, with their hands properly secured in the chamber's gloves, they must place the bag within the cutting area. | ZSH020 active |
| 5 | Once the bag is located, the operator must give the order to close the bar (Solenoid AY035 OFF) and the entrance gate of the connection tunnel (Solenoid AY033 OFF) from the HMI | ZSL020 active ZSL001 active |
| 6 | Gate seals inflate (Solenoid AY031 ON and Solenoid AY021 ON) | PSH014 active PSH015 active |
| 7 | Aspiration fan GK002 Starts | GK002 Running |
| 8 | The cutter piston is activated by the operator from the HMI. Cutter piston extends to cut bag (Solenoid AY0019 ON) | ZSH013 active |
| 9 | The product will fall into the lower hopper |  |
| 10 | The cutter piston is deactivated by the operator from the HMI. Cutter piston retracts to cut bag (Solenoid AY0019 OFF) | ZSL013 active |
| 11 | The operator must open the outlet gate (Solenoid AY0026 ON) of the cut Glove Box by command from the HMI. Before opening the gate, the seals are deflated (Solenoid AY031 OFF and Solenoid AY021 OFF) and A Timer “T_RunGK002” starts | ZSH012 active |
| 12 | To remove the empty bag or package it is necessary for the operator to raise the safety bar by activating it from the HMI (Solenoid AY035 ON). | ZSH020 active |
| 13 | Once the bar is up, the operator inserts his hands into the gloves and fold the empty bag over the first two folds (End) and then over the second (Center), as shown in the figure 2. With the package folded, the operator will enter it into the compressor through the connection tunnel. |  |
| 14 | At this point the operator should give the order to close the bar (Solenoid AY035 OFF) and the outlet gate of the cut Glove Box (Solenoid AY026 OFF) from the HMI | ZSL020 active ZSL012 active |
| 15 | The compactor pistons are activated will reduce the volume of the bag, crushing it into the continuous liner (Solenoid AY019 ON and AY018 OFF). | ZSH014 active |
| 16 | If the sensor ZSHL015 is activated and the sensor ZSH014 is not activated during compaction, it means that the compactor is full and needs to be emptied. |  |
|  | If the timer “T_RunGK002” elapse the system will Stop GK002. | GK002 Stop |
| 17 | Finally, the compactor pistons retract and ending a charging cycle (Solenoid AY019 OFF and AY018 ON). | ZSL014 active |

Figure 2: Fold the empty bag.

### CB system. Unloading from station Glove Box to Dryer/Cooler VK006

The product will be dosed by a screw feeder NC005 that will bring the powder to the heater/cooler VK006.

| Permissive list |  |  |
| --- | --- | --- |
| No emergency alarms present. Dryer/cooler VK006 not in alarm. |  |  |
| Step | Action | Completion criteria |
| 1 | Automatic Cycle Start Command (Dryer/cooler VK006) |  |
| 2 | Aspiration fan GK002 Starts | GK002 Running |
| 3 | Gate seals inflate (Solenoid AY031 ON and Solenoid AY021 ON) | PSH014 active PSH015 active |
| 4 | Inspection Hatch Closed Verification | ZSL004.VK006 active |
| 5 | Calculation of Temperature Control Values During Load (Dryer/cooler VK006) | T_Min_Loading = Oil Inlet Temperature - SET_MaxDifferenceDuringLoad T_Max_Loading = T_Min_Loading + SET_HysteresisDifferenceDuringLoad |
| 6 | VK006 system open the Load valve LP009 | ZSH018 Active |
| 7 | The hopper from Glove Box discharge valve opens LP006 | ZSH017 Active |
| 8 | Fluidization cycle start (AY001) | Fluidization cycle Running |
| 9 | The screw feeder NC005 starts at a certain speed + AY056 ON | NC005-M02-VFD Running |
| 10 | Bridge Breaker NC005 starts | NC005-M01 Running |
| 11 | Engine Startup Riboccone CMD VK006-M03-VFD ON | RUN VK006.M03.VFD active |
| 12 | Temperature Control Activation During Load (Dryer/cooler VK006) | DryerFeedSystem active |
| 13 | A timer “T_UnloadNC005” starts |  |
| 14 | Loading. | T_UnloadNC005 elapsed |
| 15 | Load Terminated | Load_Finished |
| 16 | Stop Temperature Control During Loading (Dryer/cooler VK006) | DryerFeedSystem not active |
| 17 | Once loading is completed, by the elapsing of the timer, the bridge breaker and the screw feeder will be deactivated | NC005-M01 Stopped NC005-M02-VFD Stopped |
| 18 | The hopper from Glove Box discharge valve LP006 closes | ZSL017 Active |
| 19 | VK006 system closes the Load valve LP009 | ZSL018 Active |
| 20 | Fluidization cycle stops (AY0001) and timer “T_RunGK002” starts | Fluidization cycle stops |
| 21 | If the timer “T_RunGK002” elapse the system will Stop GK002. | GK002 Stop |

### CB system. Dryer/cooler VK006 sequence.

The following sequence shows the main actions that the Dryer/Cooler performs during its process.

| Necessary conditions |  |  |
| --- | --- | --- |
| *Add alarms that you consider essential OKAWARA has not considered any* Automatic mode |  |  |
| Step | Action | Completion criterion |
| 1 | Vacuum Pump Motor Start CMD PG001-M05-VFD ON + HY013.VK006 vacuum breaker solenoid valve opening control AY013.VK006 ON + Waiting Time Vacuum Pump Startup | RUN PG001-M05-VFD active + Elapsed Vacuum Pump Start Time DLYT_VacuumPump_SwitchOn |
| 2 | LP003 Vacuum Breaker Solenoid Valve Closure AY013.VK006 OFF+ Solenoid valve opening command Cleaning with Gas from Below AY024.VK006 ON+ Vacuum Control Activation PRV012 valve controlled by differential pressure PDIT001 loop. | VacuumControlActive ON |
| 3 | Waiting time Filter cleaning | Elapsed Filter Cleaning Time DLYT_Cleaning_Filter |
| 4 | Reaching Internal Temperature Threshold | Indoor Temperature > SP_HotProduction_Temperature |
| 5 | Waiting Time Execution Process | Elapsed Process Execution Time DLYT_Production_Duration |
| 6 | Solenoid valve closing command Cleaning with Gas from Below AY024.VK006 OFF+ deactivation of Hot Oil + request signal Sending Cold Oil Request Signal + Temperature Control Activation During Process | HotOil_REQ not active + CoolOil_REQ active + Active CoolerFeedSystem |
| 7 | Reaching Internal Temperature Threshold | Indoor Temperature < SP_CoolProduction_Temperature |
| 8 | Vacuum Pump Motor Stop CMDPG001.M05.VFD OFF + Stop Vacuum Control + deactivation of Cold Oil demand signal + Temperature Control Deactivation During Process | RUN PG001-M03-VFD inactive + VacuumControlActive not active + CoolOil_REQ not active + CoolerFeedSystem not active |
| 9 | Activating Filter Cleaning Sequence + Waiting time Filter cleaning | Elapsed Filter Cleaning Time DLYT_Cleaning_Filter |
| 10 | Stop Filter Cleaning Sequence |  |
| 11 | Solenoid valve opening command Cleaning with Gas from Below AY024.VK006 ON | Internal Pressure > TH_ProductionVacuum |
| 12 | Solenoid valve closing command Cleaning with Gas from Below AY024.VK006 OFF |  |
| 13 | Stop Motor Rebound CMDVK006.M03.VFD OFF | RUN VK006-M03-VFD inactive |
| 14 | Sending Signal For Discharge Enablement Request | Discharge_ON active |

### CB system. Unloading from Dryer/cooler VK006 to VZ007

Once the heating/cooling phase is completed, the powder will be transported by the Smartjet VZ007 to the hopper TE009. The following table shows the transport loading sequence.

| Permissive list |  |  |
| --- | --- | --- |
| No High level on VZ007 (LSH002 not active) VZ007 must be available and not running. No emergency alarms present |  |  |
| Step | Action | Completion criteria |
| 1 | Sending Signal for Charge Enablement Request (VZ007). |  |
| 2 | Transporter inlet valve LP001 Opens | ZSH005 active |
| 3 | Receiving Valley Station Signal of Discharge Enablement (VK006). | Discharge_Release active |
| 4 | HY007A/B.VK006 exhaust solenoid valve opening control AY007A.VK006 ON AY007B.VK006 OFF + AirKnocker Sequence Activation (AY008.VK006) | ZSH004.VK006 not active + AirKnocker_ON active |
| 5 | Waiting time Discharge | Elapsed Discharge Time DLYT_Discharge |
| 6 | Electrovalve opening command for cleaning Blades Riboccone AY023.VK006 ON + AY028.VK006 ON + Engine Startup Riboccone CMD VK006-M03-VFD ON | RUN VK006-M03-VFD active |
| 7 | Waiting Time Cleaning Blades Riboccone | Time Cleaning Ribbon Blades spent DLYT_CleaningRibbonBlade |
| 8 | Stop Motor Rebound CMD VK006-M03-VFD OFF + AirKnocker Sequence Stop (AY008.VK006) + Solenoid valve cleaning opening control Exhaust valve gasket AY014.VK006 ON + AY015.VK006 ON + Electrovalve Cleaning Blades Riboccone closing command AY023.VK006 OFF + + AY028.VK006 OFF | RUN VK006-M03-VFD not active + AirKnocker_ON not active |
| 9 | Waiting Time Cleaning Exhaust Valve Gasket | Time spent cleaning Exhaust Valve Gasket DLYT_CleaningDischargeSeal |
| 10 | A timer “T_loadVZ007” starts |  |
| 11 | Loading. | High level of the transporter (LSH002ACTIVE) OR T_loadVZ007 elapsed |
| 12 | loading is completed, by the level activation or by the elapsing of the timer |  |
| 13 | HY007A/B.VK006 exhaust solenoid valve closing control AY007A.VK006 OFF AY007B.VK006 ON + + Stop Signal For Discharge Enablement Request | ZSH004.VK006 active + Discharge_ON not active |
| 14 | Transporter inlet valve LP370 Closes | ZSL365 Active |
| 15 | Loading completed |  |

### CB system. Convey from VZ007 to TE009

Once loading is completed, the Transporter will firstly pressurize and then convey the product to TE009.

| Permissive list |  |  |
| --- | --- | --- |
| No emergency alarms present. Loading Phase completed. No High level in TE009(). |  |  |
| Step | Action | Completion criteria |
| 1 | The Inlet valve LP001 Closes | ZSL005 Active |
| 2 | Solenoid AY005 ON to achieve the pressure set and start conveying |  |
| 3 | The transporter is now pressurizing. A timer “T_PressurizationVZ017” will start |  |
| 4 | Run Filter CA008 | CA08 Running |
| 5 | Pressure sensed at transporter PS001 Active > H level | PS001 Active > H level |
| 6 | The transporter is now in convey phase. A timer “T_conveyVZ017” start | Timer elapsed or PS001 deactivate < L level |
| 7 | The solenoid AY005 OFF |  |
| 8 | A timer “T_depressurizationVZ007” starts and timer “T_RunCA008” starts |  |
| 10 | Once PS001 deactivate < L level the system goes in standby | PIT383 < L level |
| 11 | System is in standby, if another cycle is required the transporter will carry on the loading phase and consequently the convey phase. If no more cycles are required the transporter will remain in standby mode. |  |
| 12 | If the timer “T_RunCA018” elapse the system will stop CA018. | CA018 Stop |

### CB system. Unloading from T019 to Slurry Preparation.

That finally will unload the powder to the slurry preparation system. The slurry system will send a hardwired contact to the NTE PLC in order to allow the discharge. This sequence consists of the unload of the powder stored into TE009 to the slurry preparation system (not in NTE scope). The following table shows the sequence.

| Permissive list |  |  |
| --- | --- | --- |
| The magnetic filter C011 must be available. No emergency alarms present. Slurry system available and ready to receive powder |  |  |
| Step | Action | Completion criteria |
| 1 | Slurry system send a contact to start the unload sequence |  |
| 2 | Run Filter CA008 | CA008 Running |
| 3 | The C011 discharge valve LP008 opens | ZSH011 active |
| 4 | Vibrator NO001-M01-VFD starts | NO001-M01-VFD Running |
| 5 | The C011 charge valve LP007 opens | ZSH007 active |
| 6 | Rotational unloader NC010-M01-VFD starts | NC010-M01-VFD Running |
| 7 | A timer “T_UnloadTE009” starts |  |
| 8 | Unloading. | WT001 = 0 or T_UnloadTE009 or LS001 ACTIVE |
| 9 | Once unloading is completed, by the activation or by the elapsing of the timer, the rotational unloader will be deactivated | NC010-M01-VFD Stopped |
| 10 | The C011 charge valve LP007 Closes and timer “T_RunCA008” starts | ZSL010 active |
| 11 | Stop Vibrator NO001-M01-VFD | NO001-M01-VFD Stopped |
| 12 | The C011 discharge valve LP008 Closes | ZSL011 active |
| 13 | If the timer “T_RunCA018” elapse the system will Stop CA008. | CA008 Stop |

## State 16. Completing - Almost complete

In State 16, the Dryer Hx014, Cooler VK016 and Dryer/Cooler VK006 equipment are arranged in sequence and transition to a different state to indicate to the user that their process is about to be completed.

The following tables show the idle status of the HX014 devices.

| Dryer Hx014, Cooler VK016 and Dryer/Cooler VK006 (Completing). |  |  |
| --- | --- | --- |
| TAG | Description | Position / State |
| HX014 VK016 VK006 | Current time T_Production_Time < (time less than the setpoint) setpoint to change state to almost complete SP_ Almost _ Complete | State 6 |
| HX014 VK016 VK006 | Current time T_Production_Tim => (time equal to or greater than the setpoint) setpoint to change state to almost complete SP_ Almost _ Complete | State 16 |
| HX014 VK016 VK006 | Current time T_Production_Tim = (time equal to production duration time) production duration time DLYT_Production_Duration | State 16 |
| HX014 VK016 VK006 | Unloading | State 16 |

### LFP-NCO system. Dryer HX014 Completing.

In this section, only the sequence that has the dryer in the state completing is displayed.

| Necessary conditions |  |  |
| --- | --- | --- |
| *Add alarms that you consider essential OKAWARA has not considered any* Automatic mode Current time > setpoint almost complete |  |  |
| Step | Action | Completion criterion |
| 1 | Waiting Time Execution Process | Elapsed Process Execution Time DLYT_Production_Duration |
| 2 | Vacuum Pump Motor Stop CMDPG398.M01.VFD OFF + Stop Vacuum Control | RUNPG398.M01.VFD not active + VacuumControlActive not active |
| 3 | Filter Cleaning Sequence Activation Waiting time Filter cleaning | Elapsed Filter Cleaning Time DLYT_Cleaning_Filter |
| 4 | Stop Filter Cleaning Sequence |  |
| 5 | Solenoid valve opening control Cleaning with Gas from Below HY449.HX014 AY449.HX014 ON | Internal Pressure > TH_ProductionVacuum |
| 6 | Solenoid valve closing command Cleaning with Gas from Below HY449.HX014 AY449.HX014 OFF |  |
| 7 | Stop Motor Rebound CMDHX014.M01.VFD OFF | RUNHX014.M01.VFD inactive |
| 8 | Sending Signal For Discharge Enablement Request (To VK016). | Discharge_ON active |
| 9 | Receiving Valley Station Signal of Discharge Enablement (From VK016) | Discharge_Release active |
| 10 | HY333A/B.HX014 exhaust solenoid valve opening control XY333A.HX014 ON XY333B.HX014 OFF + AirKnocker Sequence Activation | ZSH263.HX014 not active + AirKnocker_ON active |
| 11 | Waiting time Discharge | Elapsed Discharge Time DLYT_Discharge |
| 12 | Solenoid valve opening command for cleaning Blades Riboccone HY447.HX014, HY448.HX014 AY447.HX014 ON + AY448.HX014 ON + Engine Startup Riboccone CMDHX014.M01.VFD ON | RUNHX014.M01.VFD active |
| 13 | Waiting Time Cleaning Blades Riboccone | Time Cleaning Ribbon Blades spent DLYT_CleaningRibbonBlade |
| 14 | Stop Engine Rebound CMDHX014.M01.VFD OFF + AirKnocker Sequence Stop + Command opening solenoid valve cleaning gasket drain valve HY450.HX014, HY451.HX014 AY450.HX014 ON AY451.HX014 ON Electrovalve Cleaning Blades Riboccone HY447.HX014, HY448.HX014 closing command AY447.HX014 OFF + AY448.HX014 OFF | RUNHX014.M01.VFD inactive + AirKnocker_ON not active |
| 15 | Waiting Time Cleaning Exhaust Valve Gasket | Time spent cleaning Exhaust Valve Gasket DLYT_CleaningDischargeSeal |
| 16 | HY333A/B.HX014 exhaust solenoid valve closing control AY333A.HX014 OFF AY333B.HX014 ON + Stop Signal For Discharge Enablement Request | ZSH263.HX014 active + Discharge_ON not active |

### LFP-NCO system. Cooler VK016 Completing.

In this section, only the sequence that has the Cooler in the state completing is displayed.

| Necessary conditions |  |  |
| --- | --- | --- |
| *Add alarms that you consider essential OKAWARA has not considered any* Automatic mode Current time > setpoint almost complete |  |  |
| Step | Action | Completion criterion |
| 1 | Waiting Time Execution Process | Elapsed Process Execution Time DLYT_Production_Duration |
| 2 | Vacuum Pump Motor Stop CMDPG400.M01.VFD OFF + Gas Pump Solenoid Valve Closure Control HY512.VK016 AY512.VK016 ON + Stop Vacuum Control + Stop Water Pump Control | RUNPG400.M01.VFD not active + VacuumControlActive not active + WaterPumpControl not active |
| 3 | Filter Cleaning Sequence Activation Waiting time Filter cleaning | Elapsed Filter Cleaning Time DLYT_Cleaning_Filter |
| 4 | Stop Filter Cleaning Sequence |  |
| 5 | Electrovalve opening command Cleaning with Gas from Below HY454.VK016 AY454.VK016 ON | Internal Pressure > TH_ProductionVacuum |
| 6 | Solenoid valve closing command Cleaning with Gas from Below HY454.VK016 AY454.VK016 OFF |  |
| 7 | Sending Signal For Discharge Enablement Request | Discharge_ON active |
| 8 | Receiving Valley Station Signal of Discharge Enablement. | Discharge_Release active |
| 9 | Stop Motor Rebound CMDVK016.M01.VFD OFF | RUNVK016.M01.VFD inactive |
| 10 | HY302A/B.VK016 exhaust solenoid valve opening control XY320A.VK016 ON XY302B.VK016 OFF + AirKnocker Sequence Activation | ZSH298.VK016 not active + AirKnocker_ON active |
| 11 | Waiting time Discharge | Elapsed Discharge Time DLYT_Discharge |
| 12 | Electrovalve opening command for cleaning recoil blades HY452.VK016, HY453.VK016 AY452.VK016 ON + AY453.VK016 ON + Engine Startup Riboccone CMDVK016.M01.VFD ON | RUNVK016.M01.VFD active |
| 13 | Waiting Time Cleaning Blades Riboccone | Time Cleaning Ribbon Blades spent DLYT_CleaningRibbonBlade |
| 14 | Stop Motor Rebound CMDVK016.M01.VFD OFF + AirKnocker Sequence Stop + Solenoid valve cleaning opening control Exhaust Valve Gasket HY455.VK016, HY456.VK016 AY455.VK016 ON AY456.VK016 ON Electrovalve Cleaning Blades Riboccone HY452.VK016, HY453.VK016 closing command AY452.VK016 OFF + AY453.VK016 OFF | RUNVK016.M01.VFD inactive + AirKnocker_ON not active |
| 15 | Waiting Time Cleaning Exhaust Valve Gasket | Time spent cleaning Exhaust Valve Gasket DLYT_CleaningDischargeSeal |
| 16 | HY302A/B.VK016 exhaust solenoid valve closing control AY302A.VK016 OFF AY302B.VK016 ON + Stop Signal For Discharge Enablement Request | ZSH263.VK016 active + Discharge_ON not active |

### CB system. Dryer/Cooler VK006 Completing.

In this section, only the sequence that has the Dryer/Cooler in the state completing is displayed.

| Necessary conditions |  |  |
| --- | --- | --- |
| Automatic mode |  |  |
| Step | Action | Completion criterion |
| 1 | Waiting Time Execution Process | Elapsed Process Execution Time DLYT_Production_Duration |
| 2 | Solenoid valve closing command Cleaning with Gas from Below HY024.VK006 AY024.VK006 OFF+ deactivation of Hot Oil + request signal Sending Cold Oil Request Signal + Temperature Control Activation During Process | HotOil_REQ not active + CoolOil_REQ active + Active CoolerFeedSystem |
| 3 | Reaching Internal Temperature Threshold | Indoor Temperature < SP_CoolProduction_Temperature |
| 4 | Vacuum Pump Motor Stop CMDPG001.M05.VFD OFF + Stop Vacuum Control + deactivation of Cold Oil demand signal + Temperature Control Deactivation During Process | RUNPG001.M03.VFD inactive + VacuumControlActive not active + CoolOil_REQ not active + CoolerFeedSystem not active |
| 5 | Activating Filter Cleaning Sequence + Waiting time Filter cleaning | Elapsed Filter Cleaning Time DLYT_Cleaning_Filter |
| 6 | Stop Filter Cleaning Sequence |  |
| 7 | Solenoid valve opening command Cleaning with Gas from Below HY024.VK006 AY024.VK006 ON | Internal Pressure > TH_ProductionVacuum |
| 8 | Solenoid valve closing command Cleaning with Gas from Below HY024.VK006 AY024.VK006 OFF |  |
| 9 | Stop Motor Rebound CMDVK006.M03.VFD OFF | RUNVK006.M03.VFD inactive |
| 10 | Sending Signal For Discharge Enablement Request | Discharge_ON active |
| 11 | Receiving Valley Station Signal of Discharge Enablement. | Discharge_Release active |
| 12 | HY007A/B.VK006 exhaust solenoid valve opening control XY007A.VK006 ON XY007B.VK006 OFF + AirKnocker Sequence Activation | ZSH004.VK006 not active + AirKnocker_ON active |
| 13 | Waiting time Discharge | Elapsed Discharge Time DLYT_Discharge |
| 14 | Electrovalve opening command for cleaning Blades Riboccone HY023.VK006, HY028.VK006 AY023.VK006 ON + + AY028.VK006 ON + + Engine Startup Riboccone CMDVK006.M03.VFD ON | RUNVK006.M03.VFD active |
| 15 | Waiting Time Cleaning Blades Riboccone | Time Cleaning Ribbon Blades spent DLYT_CleaningRibbonBlade |
| 16 | Stop Motor Rebound CMDVK006.M03.VFD OFF + AirKnocker Sequence Stop + Solenoid valve cleaning opening control Exhaust valve gasket HY014.VK006, HY015.VK006 AY014.VK006 ON + AY015.VK006 ON + Electrovalve Cleaning Blades Riboccone HY023.VK006, HY028.VK006 closing command AY023.VK006 OFF + + AY028.VK006 OFF | RUNVK006.M03.VFD not active + AirKnocker_ON not active |
| 17 | Waiting Time Cleaning Exhaust Valve Gasket | Time spent cleaning Exhaust Valve Gasket DLYT_CleaningDischargeSeal |
| 18 | HY007A/B.VK006 exhaust solenoid valve closing control AY007A.VK006 OFF AY007B.VK006 ON + + Stop Signal For Discharge Enablement Request | ZSH004.VK006 active + Discharge_ON not active |

## State 17. Completed - Sequence completed.

In State 17, the equipment concludes its respective sequences, transitioning to an inactive state. At this point, all production information will be exchanged between the equipment, as well as between the PLC and MES system.

## State 19. E-Stop - Emergency Stop pressed.

In State 19, the equipment stops and remains in a safe state, aborting with their respective sequences when the operator presses the emergency stop or one of the critical elements goes into alarm.

### LFP-NCO system big bag unloading station NZ001.

The following table shows the Emergency Stop status of the NZ001 devices.

| Big bag unloading station NZ001 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| GK002-M01-VFD | Motor for Aspiration fan GK002 | STOP |
| AY025 | Solenoid for Vibrator solenoid valves | OFF |
| AY028 | Solenoid for The piston solenoid valve gate | OFF |
| AY097 | Solenoid for inject pressure to NZ001 | OFF |
| LP036 | Discharge Valve | CLOSED |

### LFP-NCO system Hopper TE005.

The following table shows the emergency stop status of the TE005 devices.

| Hopper TE005 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA004 | Filter command | OFF |
| AY054 | Solenoid for hooper fluidization command | OFF |
| AY055 | Solenoid for hooper fluidization command | OFF |
| AY443 | Solenoid for discharge of hoppers | OFF |
| AY056 | Solenoid for inject pressure to NC006 | OFF |
| NC006-M01 | Motor for Screw feeder NC006 | STOP |
| NC006-M02-VFD | Motor for Screw feeder NC006 | STOP |

### LFP-NCO system pneumatic transport VZ007

The following table shows the emergency stop status of the VZ007 devices.

| Transporter VZ007 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP059 | Inlet valve | CLOSED |
| LP066 | Vent valve | CLOSED |
| PRV077 | Regulator for conveying | CLOSED |
| PRV078 | Regulator for cleaning | CLOSED |
| LP088 | Discharge Valve | CLOSED |

### LFP-NCO system diverter NZ460.

The following table shows the emergency stop status of the NZ460 devices.

| Diverter NZ460 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| AY463 | Solenoid for diverted command | OFF |
| AY464 | Solenoid for straight command | OFF |
| AY466 | Solenoid to inflate diverter gasket | OFF |

### LFP-NCO system Hopper TE009.

The following table shows the idle status of the TE009 devices.

| Hopper TE009 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA008 | Filter command | OFF |
| AY133 | Solenoid for hooper fluidization command | OFF |
| AY135 | Solenoid for hooper fluidization command | OFF |
| LP145 | Discharge Valve | CLOSED |

### LFP-NCO system Milling.

The following table shows the emergency stop status of the milling devices controlled by NTE.

| Milling (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| To be defined | NTE SYSTEM In normal operation | ON |
| To be defined | NTE1 is ready to charge powder | OFF |
| To be defined | NTE1 is charging powder | OFF |
| To be defined | NTE2 requires powder | OFF |
| To be defined | NTE2 have completed the sampling of the product | OFF |
| AY475A | Solenoid for Piston extension (sampling cylinder) | OFF |
| AY475B | Solenoid for Piston retraction (sampling cylinder) | ON |
| AY476A | Solenoid for Rotation of the sampling cylinder | ON |
| AY476B | Solenoid for Rotation of sampler in rest position | OFF |
| AY473 | Solenoid for Gasket inflation | OFF |
| AY474 | Solenoid for Gasket inflation | OFF |
| LP478 | Discharge Valve | CLOSED |
| AY148A | Solenoid for NZ216 Straight position | ON |
| AY148B | Solenoid for NZ216 Reject position | OFF |

### LFP-NCO system Hopper TE011.

The following table shows the emergency stop status of the TE011 devices.

| Hopper TE011 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA010 | Filter command | OFF |
| AY171 | Solenoid for hooper fluidization command | OFF |
| AY172 | Solenoid for hooper fluidization command | OFF |
| LP175 | Discharge Valve | CLOSED |

### LFP-NCO system pneumatic transport VZ012

The following table shows the emergency stop status of the VZ012 devices.

| Transporter VZ012 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP195 | Inlet valve | CLOSED |
| LP181 | Vent valve | CLOSED |
| PRV197 | Regulator for conveying | CLOSED |
| PRV196 | Regulator for cleaning | CLOSED |
| LP138 | Discharge Valve | CLOSED |

### LFP-NCO system Dryer HX014.

The following tables show the emergency stop status of the HX014 devices.

| Dryer CA013 Filter (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA008 | Filter command | OFF |
| LP236 | Inlet valve | CLOSED |

| Dryer HX014 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP239 | Inlet valve | CLOSED |
| AY333A | Solenoid for discharge valve Open position | OFF |
| AY333B | Solenoid for discharge valve Close position | OFF |
| AY450 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY451 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY447 | Solenoid for cleaning Blades Riboccone | OFF |
| AY448 | Solenoid for cleaning Blades Riboccone | OFF |
| AY449 | Solenoid for cleaning with Gas from Below | OFF |
| AY335 | Solenoid for Air knocker ctrl sys | OFF |
| HX014-M01-VFD | Motor for rotating drive | STOP |
| AY259 | Solenoid for inject pressure to the screw NC015 | OFF |
| NC015-M01-VFD | Motor for screw conveyor or feeder | STOP |

| Dryer HX014 Vacuum and bag filter (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP222 | Pressure injection valve to the bag filter unit from VX006 | CLOSED |
| LP225 | Pressure injection valve to the bag filter unit from VX006 | CLOSED |
| AY457 | Solenoid for Outlet Bag filter auto release PDIT280>atm | OFF |
| LP356 | Outlet Bag filter vacuum breaker solenoid valve | CLOSED |
| LP357 | Vacuum breaker nitrogen line valve | CLOSED |
| LP352 | Vacuum breaker Silencer line | CLOSED |
| PRV346 | Outlet Bag filter Vacuum pressure Control | CLOSED |
| LP347 | Vacuum pressure Control nitrogen line | CLOSED |
| LP342 | Vacuum pressure Control Silencer line | CLOSED |
| PG398-M03 | Motor for Vacuum pump | STOP |

### LFP-NCO system Cooler VK016.

The following tables show the emergency stop status of the VK016 devices.

| Cooler VK016 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP264 | Inlet valve | CLOSED |
| AY302A | Solenoid for discharge valve Open position | OFF |
| AY302B | Solenoid for discharge valve Close position | OFF |
| AY455 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY456 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY452 | Solenoid for cleaning Blades Riboccone | OFF |
| AY453 | Solenoid for cleaning Blades Riboccone | OFF |
| AY454 | Solenoid for cleaning with Gas from Below | OFF |
| AY304 | Solenoid for Air knocker ctrl sys | OFF |
| VK016-M01-VFD | Motor for rotating drive | STOP |

| Cooler VK016 Vacuum and bag filter (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP269 | Pressure injection valve to the bag filter unit from VX008 | CLOSED |
| LP271 | Pressure injection valve to the bag filter unit from VX008 | CLOSED |
| AY242 | Solenoid for Outlet Bag filter auto release PDIT276>atm | OFF |
| LP322 | Outlet Bag filter vacuum breaker solenoid valve | CLOSED |
| LP323 | Vacuum breaker nitrogen line valve | CLOSED |
| LP318 | Vacuum breaker Silencer line | CLOSED |
| PRV313 | Outlet Bag filter Vacuum pressure Control | CLOSED |
| LP314 | Vacuum pressure Control nitrogen line | CLOSED |
| LP309 | Vacuum pressure Control Silencer line | CLOSED |
| PG400-M04 | Motor for Vacuum pump | STOP |
| LP401 | Inlet valve of the cooler Vacuum pump | CLOSED |

### LFP-NCO system pneumatic transport VZ017

The following table shows the emergency stop status of the VZ017 devices.

| Transporter VZ017 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP363 | Inlet valve | CLOSED |
| LP370 | Vent valve | OPEN |
| LP485 | Valve equalize pressure with VK016 | OPEN |
| AY384 | Solenoid for conveying | OFF |
| AY378 | Solenoid for cleaning | OFF |

### LFP-NCO system Hopper TE019.

The following table shows the emergency stop status of the TE019 devices.

| Hopper TE011 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA018 | Filter command | OFF |
| AY435 | Solenoid for hooper fluidization command | OFF |
| AY436 | Solenoid for hooper fluidization command | OFF |
| AY038 | Solenoid for inject pressure to NC020 | OFF |
| NC020-M01 | Motor for Screw feeder NC020 | STOP |
| NC020-M02-VFD | Motor for Screw feeder NC002 | STOP |

### LFP-NCO system magnetic filters CA021.

The following table shows the emergency stop status of the VZ021 devices.

| Magnetic filters CA021 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP422 | Inlet valve | CLOSED |
| LP426 | Discharge Valve | CLOSED |

### CB system bag unloading station Glove Box.

The following table shows the emergency stop status of the station Glove Box devices.

| CB bag unloading station Glove Box (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| AY035 | Solenoid for Bar pistons | OFF |
| AY032 | Solenoid for Prechamber pistons | OFF |
| AY033 | Solenoid for Glovebox Inlet shutter | OFF |
| AY026 | Solenoid for Glovebox Outlet shutter | OFF |
| AY031 | Solenoid for Inflatable seal Inlet | OFF |
| AY021 | Solenoid for Inflatable seal Outlet | OFF |
| AY019 | Solenoid for Glovebox knife | OFF |
| AY018 | Solenoid for Compactor pistons | OFF |
| KE002-M01-VFD | Motor for Ventilation fan | STOP |
| AY001 | Solenoid for Fluidization | OFF |
| NC005-M01 | Motor for Screw feeder NC005 | STOP |
| NC005-M02-VFD | Motor for Screw feeder NC005 | STOP |
| AY004 | Solenoid for inject pressure to NC005 | OFF |
| LP006 | Discharge Valve | CLOSED |

### CB system Dryer/Cooler VK006.

The following tables show the emergency stop status of the VK006 devices.

| Dryer/cooler VK006 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP009 | Inlet valve | CLOSED |
| AY007A | Solenoid for discharge valve Open position | OFF |
| AY007B | Solenoid for discharge valve Close position | OFF |
| AY014 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY015 | Solenoid for cleaning Exhaust valve gasket | OFF |
| AY028 | Solenoid for cleaning Blades Riboccone | OFF |
| AY023 | Solenoid for cleaning Blades Riboccone | OFF |
| AY024 | Solenoid for cleaning with Gas from Below | OFF |
| AY008 | Solenoid for Air knocker ctrl sys | OFF |
| VK016-M03-VFD | Motor for rotating drive | STOP |
| AY008 | Solenoid for inject pressure to the top of VK016 | OFF |

| Dryer/cooler VK006 Vacuum and bag filter (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP006 | Pressure injection valve to the bag filter unit | CLOSED |
| AY033 | Solenoid for Outlet Bag filter auto release PDIT001>atm | OFF |
| LP009 | Outlet Bag filter vacuum breaker solenoid valve | CLOSED |
| LP005 | Vacuum breaker nitrogen line valve | CLOSED |
| LP004 | Vacuum breaker Silencer line | CLOSED |
| PRV012 | Outlet Bag filter Vacuum pressure Control | CLOSED |
| LP003 | Vacuum pressure Control nitrogen line | CLOSED |
| LP002 | Vacuum pressure Control Silencer line | CLOSED |
| PG001-M05 | Motor for Vacuum pump | STOP |

### CB system pneumatic transport VZ007

The following table shows the emergency stop status of the VZ007 devices.

| Transporter VZ017 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| LP001 | Inlet valve | CLOSED |
| AY005 | Solenoid for conveying | OFF |

### CB system Hopper TE009.

The following table shows the emergency stop status of the TE009 devices.

| Hopper TE011 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| CA008 | Filter command | OFF |
| LP426 | Discharge Valve | CLOSED |
| NC010-M01 | Motor for rotational unloader | STOP |

### CB system magnetic filters CA011.

The following table shows the emergency stop status of the VZ021 devices.

| Magnetic filters CA021 (Emergency Stop). |  |  |
| --- | --- | --- |
| Tag | Description | Position / State |
| NO001-M01-VFD | Motor for vibrator | STOP |
| LP426 | Discharge Valve | CLOSED |

## Batching System

### LFP System Weight Batching

The system has a built in batching system which allows the operator to determine the weight of LFP/NCO to be transported through the system in the current batch. The dosing and fine dosing is originally done by using Loss in Weight of TE005, and is then compared to the transported weight of VZ007.

The dosing screw will run at 75% of full speed initially, and will continue to run until transporter VZ007 is full, or until the fine dosing weight is reached.

After the Fine Dosing Weight is reached, the dosing screw will slow to 15% to finely dose the remaining weight.  The full dosing may take several full transports before the total weight is reached.

An outline of how this is achieved is listed below:

| Necessary conditions |  |  |
| --- | --- | --- |
| Automatic mode / Semi-Automatic Mode TE005 in Standby VZ007 in Standby Operator has selected LFP from HMI |  |  |
| Step | Action | Completion criterion |
| 0 | Operator enters Total Weight | Start Button Pressed |
| 1 | Weights are downloaded to system |  |
| 2 | TE005 begins discharging to VZ007 at 75% dosing speed | ZSH070 Active OR Fine Dosing Weight Active |
| 3 | If ZSH070 Active, then TE005 discharge is completed, and the transport sequence is started |  |
|  | If Fine Dosing Weight is reached, then discharge screw will slow to 15% and continue until Total Weight is reached |  |
| 4 | VZ007 Weight Captured | VZ007 Transport Sequence Commenced |
| 5 | VZ007 Weight Captured | VZ007 Transport Sequence Completed |
| 6 | VZ007 Completed Weight subtracted from VZ007 Commence Weight and subtracted from Total Batch Weight |  |

### NMC Batching System

The batching system for NMC runs in a similar way to LFP. The mill has a maximum allowable batch size of 40kg, for this reason, multiple small batches must be run to reach the total batch weight required.

The operator is able to select NMC mode from the HMI, which will unhide the field on the HMI for the Mill Batch Size.

The batching will operate in the same manner as LFP, however the target weight for each batch will be the Mill Batch Size until the Total Batch weight has been reached.

| Necessary conditions |  |  |
| --- | --- | --- |
| Automatic mode / Semi-Automatic Mode TE005 in Standby VZ007 in Standby Operator has selected NMC from HMI |  |  |
| Step | Action | Completion criterion |
| 0 | Operator enters Total Weight |  |
| 1 | Operator enters Mill Batch Weight | Start Button Pressed |
| 2 | Weights are downloaded to system |  |
| 3 | TE005 begins discharging to VZ007 at 75% dosing speed | ZSH070 Active OR Fine Dosing Weight Active |
| 4 | If ZSH070 Active, then TE005 discharge is completed, and the transport sequence is started |  |
|  | If Fine Dosing Weight is reached, then discharge screw will slow to 15% and continue until Total Weight is reached |  |
| 5 | VZ007 Weight Captured | VZ007 Transport Sequence Commenced |
| 6 | VZ007 Weight Captured | VZ007 Transport Sequence Completed |
| 7 | VZ007 Completed Weight subtracted from VZ007 Commence Weight and subtracted from Total Batch Weight |  |

# Alarms

In this section all the possible root cause of alarms will be defined. The system mainly haves two severity of alarms, which may lead to an immediate shutdown or in a controlled shutdown.

## Immediate shutdown

Here are listed the possible causes of an Immediate shutdown

| Alarm | Description | Possible causes |
| --- | --- | --- |
| Inlet valve does not close | The control for closing of the valve is given by the PLC but the limit switch does not signal a closed valve. | - Limit switches are not correctly adjusted. - No air supply to the control solenoid valve. - Mechanical blockage of the valve caused by foreign debris. |
| Vent valve does not close |  |  |
| Discharge valve does not close |  |  |
| Inlet valve limit switches signals both open and closed | Both the closed limit switch and the open limit switch signals at the same time. |  |
| Vent valve limit switches signals both open and closed |  |  |
| Hopper discharge valve limit switches signals both open and closed |  |  |
| Inlet valve does not open | The control for closing of the valve is given by the PLC but the limit switch does not signal a closed valve. |  |
| Vent valve does not open |  |  |
| Hopper discharge valve does not open |  |  |
| Pressure transmitter out of limits | The pressure transducer of the transport vessel is out if limits | - Instrument failure |
| Emptying of the line timeout | The emptying of the line pressure is not reached during the emptying of the line before the set time has elapsed | - The material is compressed. - The line is clogged. - The “Empty line pressure setting” is too low. - The filter is clogged. |
| Depressurization timeout | The low pressure is not reached when the vent valve is opened | - The filter is clogged. - PT7035 instrument failure |
| Emergency | The emergency push-button has been pressed. All motors and auxiliary signals are cut |  |
| Auxiliaries disabled | The reset emergency push-button has not been pressed. All motors and auxiliary signals are cut. |  |
| Gasket not inflated | The command to inflate the gasket has been given and the feedback of the pressure switch is not received within a certain timer | - Gasket damaged - Pressure switch not regulated correctly - Missing air supply Wire damaged |
| Diverter not in correct position | The command to move the diverter and the relative limit switch is not received within a certain time | - Limit switches not regulated correctly - Diverter is mechanically locked - Missing air supply - Wire damaged |
| PDSH:High differential pressure active | High differential pressure over the filtering elements. Filtering elements to be changed as soon as convenient. | - Filtering elements clogged. |
| Motor trip | Motor get’s tripped | - High temperature in motor enclosure - Motor malfunction |

## Controlled shutdown

| Alarm | Description | Possible causes |
| --- | --- | --- |
| Conveying too long | The empty line unblocking- pressure is not reached during the conveying cycle before set time has elapsed and the unblocking of the line pressure has been used | - The material is compressed. - The line is clogged. - The pressure setting is too low. - The filter is clogged. |

## Warnings

It is intended as a warning an event that will be raised to the operator by HMI or MES. It will not stop the sequence, but it has to be rectified by the maintenance as soon as possible

| Alarm | Description | Possible causes |
| --- | --- | --- |
| Loading too long time | Timeout for loading the transport vessel | - Not enough product to load the transport vessel - High level transport vessel instrument failure |
| High level transport vessel and empty line in conveying | The high level of the transport vessel signals while the pressure drops below the empty line pressure in conveying | - Level switch fault - Air failure - Pilot line leaks - Pressure transmitter failure |
| Pressurization of the transport vessel too long | The transport pressure is not reached inside the given time | - Air failure - Pilot line leaks - Pressure transmitter failure - No product was loaded inside the transport vessel |

## Interlocks

The following list contains the interlocks that are considered active on each phase of the process. These interlocks will not be available if the system is in Manual Mode.

| Interlock | Cause | Effect | Value |
| --- | --- | --- | --- |
| Any Emergency pushbutton pressed | emergency pushbutton pressed | Auxiliary voltage Cut OFF | ON/OFF |
| PDIT135 Alarm or Instrument Failure | Overpressure of the Hepa Filter GC004 | Unload of LFP Big Bag to TE005 not allowed | Customizable by HMI |
| AY052 | High Pressure to the vent line of TE005 | Unload of LFP Big Bag to TE005 not allowed | ON/OFF |
| AY050 | High Pressure to the vent line of TE005 | Unload of LFP Big Bag to TE005 not allowed | ON/OFF |
| PS051 | High Pressure to the vent line of TE005 | Unload of LFP Big Bag to TE005 not allowed | ON/OFF |
| LSH039 | High level covered in TE005 | Unload of LFP Big Bag to TE005 not allowed | ON/OFF |
| PIT040 High Level Alarm or Instrument Failure | High Pressure Alarm in TE005 | Unload of LFP Big Bag to TE005 not allowed | Customizable by HMI |
| PDIT046 Alarm or Instrument Failure | High Pressure in filter CA004 | Unload of LFP Big Bag to TE005 not allowed | Customizable by HMI |
| PDIT086 High level or Instrument Failurs | Overpressure of the Hepa Filter GC003 | Unload of LFP Big Bag to TE005 not allowed | ON/OFF |
| NC006 VFD Failure | Trip or VFD Malfunction | load of transporter VZ007 not allowed | ON/OFF |
| NC006-M01 Trip Failure | TRIP or Motor failure | load of transporter VZ007 not allowed | ON/OFF |
| LSH070 | High Level sensed in Transporter VZ007 | transporter VZ007 will stop the load Phase. | ON/OFF |
| PIT075 Alarm or Instrument Failure | High pressure sensed in Transporter VZ007 | load of transporter VZ007 not allowed | ON/OFF |
| AY063 | High pressure sensed at VZ007 vent line | Transporter VZ007 goes in standby | ON/OFF |
| PSH064 | High pressure sensed at VZ007 vent line | Transporter VZ007 goes in standby | ON/OFF |
| AY065 | High pressure sensed at VZ007 vent line | Transporter VZ007 goes in standby | ON/OFF |
| diverter NZ460 not in position (no ZSH462 or ZSL461 according to the destination) | position feedback defective or diverter defective | Convey of VZ007 not allowed | ON/OFF |
| AY114 | High pressure to the vent line of TE009 | Transporter VZ007 goes in standby | ON/OFF |
| PSH115 | High pressure to the vent line of TE009 | Transporter VZ007 goes in standby | ON/OFF |
| AY116 | High pressure to the vent line of TE009 | Transporter VZ007 goes in standby | ON/OFF |
| PDIT138 Alarm or Instrument Failure | Overpressure of the Hepa Filter GC008 | Conveying to TE009 not allowed / Transporter VZ007 goes in standby | Customizable by HMI |
| PDIT117 Alarm or Instrument Failure | High Pressure in filter CA008 | Conveying to TE009 not allowed / Transporter VZ007 goes in standby | Customizable by HMI |
| PIT124 Alarm or Instrument Failure | High Pressure Alarm in TE009 | Conveying to TE009 not allowed /Transporter VZ007 goes in standby | Customizable by HMI |
| AY114 | High pressure to the vent line of TE009 | TE009 goes into standby | ON/OFF |
| PSH115 | High pressure to the vent line of TE009 | TE009 goes into standby | ON/OFF |
| AY116 | High pressure to the vent line of TE009 | TE009 goes into standby | ON/OFF |
| PDIT138 Alarm or Instrument Failure | Overpressure of the Hepa Filter GC008 | TE009 goes into standby | Customizable by HMI |
| PDIT117 Alarm or Instrument Failure | High Pressure in filter CA008 | TE009 goes into standby | Customizable by HMI |
| PIT124 Alarm or Instrument Failure | High Pressure Alarm in TE009 | TE009 goes into standby | Customizable by HMI |
| Diverter NZ216 not in position (no ZSH149 or ZSL150 according to the destination) | position feedback defective or diverter defective | Conveying to TE011 not allowed / Transporter VZ007 goes in standby | ON/OFF |
| PDIT140 Alarm or Instrument Failure | Overpressure of the Hepa Filter GC010 | Conveying to TE011 not allowed / Transporter VZ007 goes in standby | Customizable by HMI |
| PDIT157 High Alarm or Instrument Failure | High Pressure in filter CA010 | Conveying to TE011 not allowed / Transporter VZ007 goes in standby | Customizable by HMI |
| PIT162 Alarm or Instrument Failure | High Pressure in TE011 | Conveying to TE011 not allowed / Transporter VZ007 goes in standby | Customizable by HMI |
| AY160 | High pressure sensed at TE011 vent line | Conveying to TE011 not allowed / Transporter VZ007 goes in standby | ON/OFF |
| PSH159 | High pressure sensed at TE011 vent line | Conveying to TE011 not allowed / Transporter VZ007 goes in standby | ON/OFF |
| AY158 | High pressure sensed at TE011 vent line | Conveying to TE011 not allowed / Transporter VZ007 goes in standby | ON/OFF |
| PDIT140 Alarm or Instrument Failure | Overpressure of the Hepa Filter GC010 | TE011 goes into standby | Customizable by HMI |
| PDIT157 High Alarm or Instrument Failure | High Pressure in filter CA010 | TE011 goes into standby | Customizable by HMI |
| PIT162 Alarm or Instrument Failure | High Pressure in TE011 | TE011 goes into standby | Customizable by HMI |
| AY160 | High pressure sensed at TE011 vent line | TE011 goes into standby | ON/OFF |
| PSH159 | High pressure sensed at TE011 vent line | TE011 goes into standby | ON/OFF |
| AY158 | High pressure sensed at TE011 vent line | TE011 goes into standby | ON/OFF |
| AY178 | High pressure sensed at VZ012 vent line | Transporter VZ012 goes in standby | ON/OFF |
| PSH179 | High pressure sensed at VZ012 vent line | Transporter VZ012 goes in standby | ON/OFF |
| AY180 | High pressure sensed at VZ012 vent line | Transporter VZ012 goes in standby | ON/OFF |
| PIT194 High Alarm or Instrument Failure | High pressure sensed at VZ012 | Load of transporter VZ012 not allowed | Customizable by HMI |
| LSH183 | High Level covered in VZ012 | Load of transporter VZ012 not allowed | ON/OFF |
| PDIT141 Alarm or Instrument Failure | Overpressure of the Hepa Filter GC013 | Convey of VZ012 not allowed / HX014 goes in standby | Customizable by HMI |
| PDIT234 Alarm or Instrument Failure | High pressure in filter CA013 | Convey of VZ012 not allowed / HX014 goes in standby | Customizable by HMI |
| TIT233 Alarm or Instrument Failure | High temperature in filter CA013 | Convey of VZ012 not allowed / HX014 goes in standby | Customizable by HMI |
| PDIT280 Alarm or Instrument Failure | High pressure sensed at HX014 | Convey of VZ012 not allowed / HX014 goes in standby | Customizable by HMI |
| TIT246 High Threshold Alarm or Instrument Failure | High temperature sensed at HX014 | Convey of VZ012 not allowed / HX014 goes in standby | Customizable by HMI |
| LSH142 | High level covered in filter CA013 | Convey of VZ012 not allowed / HX014 goes in standby | Customizable by HMI |
| AIT281 Alarm or Instrument Failure | High Oxigen concentration sensed | Convey of VZ012 not allowed / HX014 goes in standby | Customizable by HMI |
| ZIT346 Alarm or Failure | position feedback defective or regulator valve defective | Convey of VZ012 not allowed / HX014 goes in standby | ON/OFF |
| ZSL262 not active | Inspection hatch open in HX014 | Convey of VZ012 not allowed / HX014 goes in standby | ON/OFF |
| NC015 VFD Failure | Trip or VFD Malfunction | Unload of HX014 not allowed | ON/OFF |
| HX014-M01 VFD Failure | Trip or VFD Malfunction | Convey of VZ012 not allowed / HX014 goes in standby | ON/OFF |
| PDIT276 Alarm or Insturment Failure | High pressure sensed at VK016 | Load of VK016 not allowed | Customizable by HMI |
| TIT285 High Threshold Alarm or Instrument Failure | High Temperature sensed at VK016 | Load of VK016 not allowed | Customizable by HMI |
| AIT277 Alarm or instrument Failure | High Oxigen concentration sensed | VK016 goes in standby | Customizable by HMI |
| ZSL292 not active | Inspection hatch open in VK016 | VK016 goes in standby | ON/OFF |
| ZSH298 not active | discharge hatch not closed | Load of VK016 not allowed | ON/OFF |
| ZSH263 not active | discharge hatch not closed | Load of HX014 not allowed | ON/OFF |
| ZIT495 Alarm or Failure | position feedback defective or regulator valve defective | Load of VK016 not allowed / Vk016 goes in standby | ON/OFF |
| VK016-M01 VFD Failure | Trip or VFD Malfunction | Load of VK016 not allowed / Vk016 goes in standby | ON/OFF |
| PG398-M03 VFD Failure | Trip or VFD Malfunction | HX014 goes in standby | ON/OFF |
| TSH498 Alarm | OverHeating of Vacuum Pump PG398 | HX014 goes in standby | ON/OFF |
| PIT383 High Alarm or Instrument Failure | High pressure sensed at VZ017 | Load of transporter VZ017 not allowed | Customizable by HMI |
| AY367 | High pressure sensed at VZ017 vent line | Transporter VZ017 goes in standby | ON/OFF |
| PSH368 | High pressure sensed at VZ017 vent line | Transporter VZ017 goes in standby | ON/OFF |
| AY369 | High pressure sensed at VZ017 vent line | Transporter VZ017 goes in standby | ON/OFF |
| LSH372 | High Level covered in VZ017 | Load of transporter VZ017 not allowed | ON/OFF |
| AY408 | High pressure sensed at TE019 vent line | Transporter VZ017 goes in standby | ON/OFF |
| PSH409 | High pressure sensed at TE019 vent line | Transporter VZ017 goes in standby | ON/OFF |
| AY410 | High pressure sensed at TE019 vent line | Transporter VZ017 goes in standby | ON/OFF |
| PDIT209 High alarm or instrument failure | Overpressure of the Hepa Filter GC018 | Convey of VZ017 not allowed | Customizable by HMI |
| PDIT413 Alarm of Instrument failure | High pressure in filter CA018 | Convey of VZ017 not allowed | Customizable by HMI |
| LSH415 | High Level covered in TE019 | Convey of VZ017 not allowed | ON/OFF |
| PIT416 Alarm or Instrument Failure | High pressure sensed in TE019 | Convey of VZ017 not allowed | Customizable by HMI |
| NC020 VFD Failure | Trip or VFD Malfunction | Unload of TE019 not allowed | ON/OFF |
| NC020-M01 Failure | TRIP or Motor failure | Unload of TE019 not allowed | ON/OFF |
| AY446 | Magnetic Separator CA021 Inspection Hatrch open | Unload of TE019 not allowed | ON/OFF |
| PG400-M04 VFD Failure | Trip or VFD Malfunction | VK016 goes in standby | ON/OFF |
| PIT128 High Alarm or Instrument Failure | High Pressure sensed at NZ001 | Unload of LFP Big Bag to TE005 not allowed | Customizable by HMI |
| PDIT003 High Alarm or Instrument Failure | Overpressure of the Hepa Filter GC002 | Unload of CB Bag not allowed | Customizable by HMI |
| 1002-PIT001 High Alarm or Instrument Failure | High Pressure sensed at CX001 | auxiliary voltage Cut OFF - SAFE | Customizable by HMI |
| 1002-TIT001 High Alarm or Instrument Failure | High Temperature sensed at CX001 | Unload of CB Bag not allowed | Customizable by HMI |
| GK002 VFD Failure | Trip or VFD Malfunction | Unload of CB Bag not allowed | ON/OFF |
| ZSL016 Not Active | Compactor Door Open | Unload of CB Bag not allowed | ON/OFF |
| LS001 | High Level covered in CX001 | Unload of CB Bag not allowed | ON/OFF |
| NC005 VFD Failure | Trip or VFD Malfunction | Load of VK006 not allowed | ON/OFF |
| NC005-M01 Failure | TRIP or Motor failure | Load of VK006 not allowed | ON/OFF |
| AIT001 Alarm or Instrument Failure | High Oxigen concentration sensed | Load of VK006 not allowed/VK006 goes in standby | Customizable by HMI |
| PDIT001 Alarm or Instrument Failure | High pressure sensed at VK006 | Load of VK006 not allowed/VK006 goes in standby | Customizable by HMI |
| TIT002 Alarm or Instrument Failure | High temperature sensed at VK006 | Load of VK006 not allowed/VK006 goes in standby | Customizable by HMI |
| ZIT001 Alarm or Failure | position feedback defective or regulator valve defective | Load of VK006 not allowed / Vk006 goes in standby | Customizable by HMI |
| ZSL004 | Inspection hatch open in VK006 | VK006 goes in standby | ON/OFF |
| ZSH004 | discharge hatch not closed | Load of VK006 not allowed | ON/OFF |
| PSH001 | High pressure sensed at VZ007 | Load of VZ007 not allowed | ON/OFF |
| AY036 | High pressure sensed at VZ007 vent line | VZ007 goes in standby | ON/OFF |
| PSH003 | High pressure sensed at VZ007 vent line | VZ007 goes in standby | ON/OFF |
| AY034 | High pressure sensed at VZ007 vent line | VZ007 goes in standby | ON/OFF |
| LSH002 | High level covered at VZ007 | Load of VZ007 not allowed | ON/OFF |
| PG001-M05 VFD Failure | Trip or VFD Malfunction | VK006 goes in standby | ON/OFF |
| PDIT004 Alarm or Instrument Failure | Overpressure of the Hepa filter GC002 | Convey of VZ007 not allowed | Customizable by HMI |
| PDIT002 Alarm or Instrument Failure | High pressure sensed at CA008 | Convey of VZ007 not allowed | Customizable by HMI |
| PIT002 Alarm or Instrument Failure | High pressure sensed at TE009 | Convey of VZ007 not allowed | Customizable by HMI |
| LSH003 | High level covered at TE009 | Convey of VZ007 not allowed | ON/OFF |
| NC010 VFD Failure | TRIP or Motor failure | Unload of TE009 not allowed | ON/OFF |
| AY021* | Magnetic Separator CA011 Inspection Hatrch open | Unload of TE009 not allowed. Stop circle feeder and close 1002-AY020 | ON/OFF |
| CP Request Powder (Signal from NCE) | Mill is ready to receive powder | Unload to Mill allowed | ON/OFF |
| CP in Normal operation (Signal from NCE) | Mill is not in alarm and is available | Unload to Mill allowed | ON/OFF |
| Ross General Alarm or E-Stop Pressed | General Alarm or E-Stop Pressed in Ross system(LFP/NMC) | unloading not allowed | ON/OFF |
| Ross General Alarm or E-Stop Pressed | General Alarm or E-Stop Pressed in Ross system(CB) | unloading not allowed | ON/OFF |
| 1002-ZSL005 | Smartjet VZ006 Load valve | 1002-AY005 will not get activated or will deactivate if in cycle | ON/OFF |
| 1002-ZSL010 | hopper TE007 valves are not closed | 1002-AY005 will not get activated or will deactivate if in cycle | ON/OFF |
| 1001-ZSL424 | 1001-TE017 dischage valve are not closed | Transporter pressurization 1001-VX017 will not get activated or will deactivate if in cycle | ON/OFF |
| 1002-CX001 Bar | CX001 Bar not closed | Knife activation not allowed | ON/OFF |

# Process Settings and Alarm Settings

In this section will be covered the process settings and the alarm settings that can be modified into the HMI on the dedicated pages. This section will be divided according to the equipment

## Transporter Setting

Here below are listed the transporter settings. These are valid for the transporters VZ007, VZ012 and VZ017 and will be managed individually for each transporter.

### Process settings

| Setting | Use | Range |
| --- | --- | --- |
| Timer_Delay_Open_Discharge | Delay timer for opening discharge valve during loading | 1 … 5 sec |
| Timer_Delay_Close_Inlet | Delay timer for closing inlet valve during loading | 1 … 5 sec |
| Timer_Delay_Start_Conv | Delay timer before starting conveying cycle | 1 … 5 sec |
| Timer_Cleaning_Line | Time for the cleaning. solenoid is on during the cleaning of the line | 0 … 600 sec |
| Low_pressure | Low pressure inside the transport vessel | 0 … 0.2 bar |
| High_pressure | Pressurization threshold for transport vessel | 0.4 … 1.2 bar |

### Alarm Settings

| Setting | Use | Range |
| --- | --- | --- |
| Timer_Loading_Too_Long | Timeout for loading the transport vessel | 10-600 sec |
| Timer_Pressurization_Too _Long | Timeout for pressurizing the vessel | 10-600 sec |
| Timer_Empty_Line | Timer for emptying the line at the end of the automatic cycle | 10 … 600 sec |
| Timer_Convey_Too_Long | Timeout for the conveying cycle | 10-600 sec |
| Timer_Depressurization_ Too_Long | Timeout for depressurizing the vessel with the vent valve open | 0-600 sec |

## Diverter Settings

| Setting | Use | Range |
| --- | --- | --- |
| Delay inflation of the gasket | Timer for inflation of the gasket | 1 – 10 sec |
| Delay feedback diverter | Timer for receiving feedback of diverter limit switch | 1 – 10 sec |

## Fluidization PAD settings

| Setting | Use | Range |
| --- | --- | --- |
| Timer_Fluid_ON | Timer fluidization ON | 1 – 10 sec |
| Timer_Fluid_OFF | Timer fluidization OFF | 1 – 10 sec |
| Number of cycles | Number of fluidization cycles | 1… 20 |

## Dryer, Cooler and Dryer/Cooler Settings

In this table are described the settings foreseen for the above mentioned Machines. All these are available and configurable in HMI

| Setting | Description | Range |
| --- | --- | --- |
| SET_InletOilTemperature | Minimum Inlet Temperature threshold of the Ribocone Oil to enable the start of the sequence | 250 – 290 °C |
| SET_MaxDifferenceDuringLoad | Maximum difference from the Initial Oil Inlet Temperature of the Internal Temperature of the Ribocone during the Loading phase | 0 – 120 °C |
| SET_HysteresisDifferenceDuringLoad | control temperature hysteresis DryerFeedSystem | 0 – 20 °C |
| T_Min_Loading | Minimum temperature threshold inside the Riboccone during the loading phase. When this threshold is exceeded, the load consent is removed. | evaluated |
| T_Max_Loading | Reset threshold of the DryerFeedSystem control. When this threshold is exceeded, the load consent is reactivated. | evaluated |
| DLYT_VacuumPump_SwitchOn | Opening time of the vacuum breaker valve during the start of the vacuum pump. | 0 – 60 sec |
| SP_Production_Temperature | Production threshold of the temperature inside the Ribocone. (Valid for Dryer and Cooler) | 0 – 290 °C |
| SP_HotProduction_Temperature | Warm Production Threshold of the temperature inside the Ribocone. (Valid only for Dryer/Cooler) | 0 – 290 °C |
| SP_CoolProduction_Temperature | Cold Production Threshold of the temperature inside the Riboccone. (Valid only for Dryer/Cooler) | 0 – 290 °C |
| DLYT_Cleaning_Filter | Filter cleaning time at the end of the load. | 0 – 60 sec |
| TH_ProductionVacuum | Maximum admissible vacuum threshold during the vacuum control phase. | 0 – 10 mbar |
| DLYT_Discharge | Time to unload the Riboccone. | 0 – 60 min |
| DLYT_CleaningRibbonBlade | Time for cleaning the blades of the Ribocone. | 0 – 60 sec |
| DLYT_AirKnocker_On | Opening time of the Air Knocker valve during its working phase. | 0 – 400 msec |
| DLYT_AirKnocker_Off | Closing time of the Air Knocker valve during its working phase. | 0 – 30 sec |
| SET_ProportionalBand | Parameter of the Proportional Band of the Vacuum PID Control | 0 – 300 °C |
| SET_IntegralTime | Vacuum PID Control Integral Time Parameter | 0 – 60 sec |
| SET_DerivativeTime | Derivative time parameter of vacuum PID control | 0 – 60 sec |

## Dryer, Cooler and Dryer/Cooler Alarms Settings.

In this chart are listed the Alarm settings valid for all machines.

| Setting | Description | Range |
| --- | --- | --- |
| Oxygen threshold TH_Oxygen | Maximum permissible threshold of the oxygen concentration | 0 – 10 % |
| Oxygen Timer DLYTA_HighOxygenLevel | When the Oxygen Value Threshold is exceeded, the following timer will start. If at the end of the Timer the PLC detects a still high concentration of Oxygen, the alarm will be generated. | 0 – 5 sec |
| High Temperature Threshold TH_MaxProductionTemperature | Maximum allowable threshold for the Ribocone's internal temperature | 0 – 300 °C |
| Delay High Temperature DLYTA_HighTemperature | When the Limit Threshold of the Temperature value is exceeded, the following timer will start. If at the end of the Timer the PLC detects a still high internal temperature, the alarm will be generated. | 0 – 5 sec |
| Vacuum threshold TH_MaxLowVacuum | Maximum admissible pressure level threshold (Maximum low vacuum level) | 0 – 30 mbar |
| Delay Vacuum Threshold DLYTA_VacuumLoss | When the vacuum value limit threshold is exceeded, the following timer will be started. If at the end of the Timer the PLC detects a still high vacuum level, the alarm will be generated. | 0 – 5 sec |
| Production time too long | During the production phase of reaching the production temperature, the following timer is started. If at the end of the Timer the PLC does not find the production reached, the alarm will be generated. | 0 – 3 h |
| Vacuum Pump Water Circuit Flow Low Level Time | When checking the water pump, the following timer is started when there is no flow. If at the end of the Timer the PLC does not receive the flow level signal, the alarm is generated. |  |
| Long Fill Time | When checking the water pump, the following timer is started when there is no flow and no signal for the closed filling valve. If at the end of the Timer the PLC does not receive the flow level signal or the filling valve closed, the alarm is generated. |  |
| High Water Temperature Time | During the control of the water pump, the following timer is started when there is no water temperature signal. If at the end of the Timer the PLC does not receive the low water temperature signal, the alarm is generated. |  |

## Dryer, Cooler and Dryer/Cooler Alarms

In this section are listed the alarms that may possibly rise during the sequences.

| Alarm | Description | Possible Root Cause |
| --- | --- | --- |
| High level Oxygen Alarm | The Oxygen level detected is too High for too long a time. The execution of the automatic cycle will be interrupted and the Stand-By conditions of the valves and motors will be restored | - Oxygen signal error - Vacuum pump not running - Vacuum control not active - Vacuum control parameterized incorrectly - Proportional valve blocked - Loss in the carpentry |
| High Temperature Alarm | The internal temperature of the Ribocone is too high for too long time. The execution of the automatic cycle will be interrupted and the Stand-By conditions of the valves and motors will be restored | • Temperature sensor signal error • Inlet Oil temperature control problem |
| Vacuum Loss Alarm | The vacuum inside the Ribocone is too high for too long. The execution of the automatic cycle will be interrupted and the Stand-By conditions of the valves and motors will be restored | • Oxygen signal error • Vacuum pump not running • Vacuum control not active • Vacuum control parameterized incorrectly • Proportional valve blocked Leak in carpentry |
| Production time too long | The production time is too long. | • Temperature sensor signal error • Inlet Oil temperature control problem • Incorrect configuration of the alarm timer |
| Vacuum Pump Water Circuit Flow Low Level Alarm | The Flow level of the Vacuum Pump water loop is low for too long |  |
| Long Fill Time Alarm | The filling time with the water circuit filling valve is too long |  |
| High Water Temperature Alarm | The temperature of the vacuum pump water circuit is high. |  |